# U i

o; tflnd k'l:tsf  ;+3Lo cfly{s ;xof]u tyf ljsf; dGqfno, (BMZ) hd{gL åf/f Deutsche Gesellschaft für Internationale Zusammenarbeit (GIZ) GmbH dfkm{t sf]ifdf lgd{f0f ul/Psf] xf] .

k|sfzg ug]{ ;+:yf

g]kfn ;/sf/
pmhf{, hn;|f]t tyf l;+rfO dGqfno
j}slNks pmhf{ k|j4{g s]G›
tfxfrn, kf]:6 aS; g+M !$#^$, sf7df8f}F, g]kfn
kmf]gM ±(&&–!–$%(*)!#÷$%(*)!$
Od]nM info@aepc.gov.np
j]aM www.aepc.gov.np

;xof]uL ;+:yf

hd{g cGt/f{li6«o ljsf; ;+:yf

k|df];g ckm ;f]nf/ 6]Sgf]nf]hL km/ Osf]gf]lds l8Enf]kd]G6 -kf]:6]8_
/fli6«o k|s[lt ;+/If0f sf]if ejg, v'dn6f/, nlntk'/, g]kfn

kf]i6 aS; gM !$%&, sf7df8f}+, g]kfn

6]lnkmf]g: ±(&&–!–$%(*)!#÷$%(*)!$

Od]nM posted@giz.de

;+of]hs ;+:yf
OlG6u|];g pDj]N6 P08 OghL{ lhPdljPr, hd{gL

n]vsM ljsf; pk|]tL, /f]lx0fL v]MF

;dLIfsM lgk'0f /]UdL, r'dg afa' >]i7, ;lbR5f Gof}kfg]

;+kfbgM /f]lx0fL v]MF, r'dg afa' >]i7, ;lbR5f Gof}kfg]

;+of]hgM /f]lx0fL v]MF

n]cfp6M /f]lx0fL v]MF

se/ kmf]6f]M km]lnS; lg6\h

kl/of]hgf k|d'vM km]lnS; lg6\h

# c:jLs/0f

z}lIfs p2]Zosf nflu tof/ kfl/Psf ;f}o{ /mkm6k ck/]6/sf]] of] tflnd k'l:tsfdf ljleGg >f]taf6 ;"rgf tyf hfgsf/Lx? ;fef/ ul/Psf] 5 . gfkmf sdfpg] p4]Zosf] nflu of] kxn ul/Psf] xf]Og .

k|sfzsn] o; k|sfzgdf k|of]u ul/Psf s'g} klg lrq0fx¿sf] nflu :jfldTj bfjL ub}{g . lrq0fsf] ;|f]t klxrfg ug{ Wofg lbOPsf] 5 . t/, ;a}sf] klxrfg x'g ;s]sf] 5}g . >]o gePsfx¿ ljleGg ;fj{hlgs cgnfOg ;|f]tx¿af6 k|fKt ul/Psf lyP hf] ;xL k|ltlnlk clwsf/ dflnsx¿ x'g\ .

l8;]Da/ @)@$

|  | g]kfn ;/sf/ pmhf{, hn;¦f]t tyf l;+rfO dGqfno j}slNks pmhf{ k¦j4{g s]G› g]kfndf gjLs/0fLo ph{fnfO{ d'nwf/df Nofpg] |  |
| --- | --- | --- |

ck/]6/ tflnd k'l:tsf

;f}o{ ¿km6k k|0ffnLx?

ljifo ;"rL

!= k|ljlwsf] kl/ro	1

!=!  ;f}o{ lkeL Kofgn (Solar PV panel)	2

!=@  Aof6«L (Battery)	3

!=#  OGe6{/ (Inverter)	5

!=$  O{ghL{ ld6/ (Energy meter)	6

!=%  TOD ld6/	6

!=^  cGo ljB'tLo / h8fgsf ;fdfgx¿ (Other electrical and installation components)	7

!=^=!  ;f}o{ Kofgn / :6«Sr/ (Solar panel and structure)	7

!=^=@  ljt/0f afs; (Distribution box)	7

!=^=#  c6f]d]l6s 6«fG;km/ l:jr (Automatic Transfer Switch)	9

!=^=$  tf/x? (Wires and cables)	10

!=^=%  cly{ª÷u|fplG8ª\u lk6 (Earthing/Grounding pit)	10

@= On]S6«f]d]sflgsn pks/0fx¿sf] dd{t / ;kmfO{	11

@=!  ;f}o{ Kofgnsf] dd{t / ;kmfO{	11

@=!=!  ;f}o{ Kofgnsf] dd{t / ;kmf ug]{ k|lqmof	11

@=!=@  ;f}o{ Kofgnsf] ef]N6]h / s/]G6 hfFRg] k|lqmof	12

@=!=#  8Ll; 8Lla aS; (DC DB box) hfFRg] k|lqmof	13

@=!=$  Surge Protection Device (SPD) km]g{] k|lqmof	13

@=@  Aof6«L :6f]/]h (Battery storage) dd{t / hfFRg] k|lqmof	14

@=#  OGe6{/ hfFRg] k|lqmof	18

@=$  ACDB dd{t;Def/ hfFRg] k|lqmof	22

@=%  ATS (Automatic Transfer Switch) hfFRg] k|lqmof	24

@=^  ;'/Iff pks/0f (Protection devices) sf] hfFr	26

@=^  TOD ld6/sf] lg/LIf0f hfFr	30

@=&  SRT k|0ffnLdf ck/]6/sf] e"ldsf / lhDd]jf/L (Operator roles and responsibilities)	32

#= ;f}o{ ¿km6k nul;6	34

# != k|ljlwsf] kl/ro

;f}o{ ¿km6k (Solar rooftop) k|ljlw eg]sf] ;f}o{ pmhf{ pTkfbg ug]{ ;]6ck xf] . o; k|ljlwdf ejgx¿sf] 5tdf ;f}o{ Kofgnx¿ h8fg ul/G5 h;n] ;"o{sf] ls/0fnfO{ lah'nLdf ¿kfGt/0f ub{5 . ljz]if u/L 7"nf ;f}o{ kmfd{x¿sf] nflu ;Lldt hldg ePsf] If]qx¿df 5fgfdf ;f}o{ Kofgn /fvL pmhf{ pTkfbg ug{ of] k|0ffnL pkof]uL x'G5 . o;/L k|fKt x'g] phf{n] k/Dk/fut pmhf{sf] ;|f]tx¿ h:t} sf7, t]n cflbaf6 k|fKt x'g] phf{df lge{/tf sd ug{, jftfj/0fdf kg]{ c;/ sd ug{ tyf lah'nLsf] lan 36fpg d2t ub{5 .

;f}o{ ¿km6k k|ljlwn] s;/L sfd ub{5 <

;f}o{ ¿km6k k|0ffnLdf d'Votof ;f}o{ Kofgn, Oge6{/ / afO{-l8/]Szgn ld6/ (Bi-directional meter) x'G5 . 5fgfdf h8fg ul/Psf] ;f}o{ Kofgnx¿n] ;"o{sf] ljls/0fnfO{ l8=;L=s/]06 (DC current)df ¿kfGt/0f ub{5 . OGe6{/n] ;f}o{ Kofgnaf6 pTkGg x'g] l8=;L= s/]06nfO{ 3/ jf lu|8df k|of]usf] nflu pko'Qm x'g] P=;L= s/]06 (AC current) df ¿kfGt/0f ub{5 . o; k|0ffnLn]] ;f]nf/ Kofgnx¿åf/f pTkGg ePsf] ah'nL k|0ffnL h8fg u/]sf] ejgdf k|of]u u/L a9L ePsf] lah'nLnfO{ lu|8df k7fpg d2t ub{5 . o;/L ;f}o{ ¿km6k k|ljlwaf6 lu|8df uPsf] lah'nLsf] /sd lu|8af6 3/df NofOPsf] lah'nLsf] /sddf s6\6f u/L afFls /sd dfq ltg{ ;lsG5 .

g]kfndf xfn ;f}o{ ¿km6k k|ljlw nf]slk|o x'Fb} uPsf] 5 . ljz]if u/L lu|8af6 af/Daf/ lah'nL sfl6g] cj:yfdf 3/, sfof{no / ;+:yfx¿sf] nflu of] pTs[i6 ljsNksf] ?kdf x]g{ ;lsG5 .

*lah'nL gePsf] ;dodf ;f}o{ ?k6k k|0ffnLsf] k|of]u ug{' k/] OGe6{/ / Aof6«L (Battery inverter) k|of]u x'G5 .

lrq !: ;f}o{ ¿km6k k|0ffnL

## !=!  ;f}o{ lkeL Kofgn (Solar PV panel)

;f}o{ ;]n (Solar cell), jf kmf]6f]ef]N6flos (lkeL) PV ;]nn] ;"o{af6 ljls/l0ft pmhf{nfO{ lah'nLdf ?kfGt/0f ub{5 . o;sf] ljB'tLo ljz]iftfx?, h:t} s/]G6, ef]N6]h jf cj/f]w, ;f}o{ k|sfz cg';f/ leGg x'G5g\ . w]/} ;f}o{ ;]nx?nfO{ hf]8] kl5 To;nfO{ ;f}o{ Kofgn eg]/ lrlgG5 .

lrq @: ;f}o{ lkeL ;]n, Kofgn, / P/] [;|f]t: IWMI–;f}o{ l;+rfO{ kDkx?af/] k|lzIf0f lgb]{lzsf @)*)]

## !=@  Aof6«L (Battery)

Aof6«L :6f]/]h (Battery storage) Pp6f o:tf] On]S6«f]d]sflgsn ;+oGq (Electromechanical mechanism) xf], h;n] /f;folgs pmhf{nfO{ hDdf u/L cfjZostf cg';f/ ljB't pmhf{df kl/0ft u/L ljB't k|jfx u/fpF5 . hj Aof6«L s'g} On]lS6«sn nf]8 (Electrical load) ;+u hf]l8G5, To;j]nf Aof6«L leq /f;folgs k|lqmof ;'? x'G5 / wgfTds Kn]6af6 C0ffTds Kn]6lt/ s/]G6 (Current from positive to negative plate) axg ;'? x'G5 . Aof6«Ldf lbpF;f] e/L ;f]nf/ kftfaf6 pTkfbg ePsf] s/]G6 jf ljB'lto zlQm hDdf x'G5 . Aof6«Ldf zlQm hDdf ug]{ k|lqmofnfO{ rflh{Ë (Charging) elgG5 . a]n'sLkv Aof6«Ldf hDdf ePsf] zlQm aQL afNg jf On]lS6«sn nf]8df k|of]u x'G5 / of] k|s[ofnfO{ l8:rflh{Ë (Discharging) elgG5 .

Aof6«Lx? w]/} k|sf/sf x'G5g, h:t}:

| - nL8-Pl;8 Aof6«Lx¿ (Lead-acid batteries) |  |  |
| --- | --- | --- |
| nL8-Pl;8 Aof6«Lx¿ (Lead-acid batteries) df ln8 Kn]6x¿ (Lead plates) nfO{ On]S6«f]nfO6 (Electrolyte) df 8'afPsf] x'G5, h'g ;]k/]6;{ (Separators) åf/f cnu ul/Psf] x'G5 . On]S6«f]nfO6sf] :t/nfO{ lgoldt ¿kdf lgl/If0f ug{' k5{ . o:tf] Aof6«Lx¿df ;do ;dodf l8l:6N8 kfgL (Distilled water) /fVg] ug'{ k5{ . | lrq $: nL8-Pl;8 Aof6«L |  |
| - eNe /]u'n]6]8 nL8-Pl;8 Aof6«Lx¿ (Valve regulated lead-acid batteries) |  |  |
| eNe /]u'n]6]8 ln8 Pl;8 Aof6«Lx¿ (Valve regulated lead-acid batteries) nfO{ ;fdfGotof l;n ul/Psf] ln8 Pl;8 (Lead-acid) Aof6«Lx¿ eg]/ lrlgG5 . o;df kfgL jf On]S6«f]nfO6 (Electrolyte) hfFrx¿ cfjZos kb{}g / dd{t ;Def/sf] cfjZostf kb{}g . | lrq %: eN\e /]u'n]6]8 ln8 Pl;8 Aof6«L |  |
| - lnlyod-cfog Aof6«Lx¿ (Lithium-ion batteries) |  |  |
| lnlyod cfog Aof6«Lx¿sf] (Lithium-ion batteries) On]S6«f]8x?, sd jhg ePsf] lnlyod / sfa{gaf6 ag]sf x'G5g\ . oL Aof6«Lx? k/Dk/fut\ ln8– Pl;8 Aof6«Lx?sf] t'ngfdf pRr pmhf{ 3gTj / pRr sfo{Ifdtf x'G5 / o;sf] cfo' bz jif{eGbf dfly x'G5 . ln8 Pl;8eGbf t'ngfTds ?kdf a9L dxFuf] x'g' lnlyod cfof]g Aof6«Lsf] k¦d'v a]kmfObf xf] . |  | lrq ^: lnlyod cfog Aof6«L |  |

Aof6«Lsf] h8fg k|s[of:

- l;/Lh h8fg (Series connection) l;/Lh h8fgdf, k¦To]s Aof6«L a}+ssf] ef]N6]h a9fpg Aof6«Lx? l;l/hdf h8fg ul/G5 . klxnf] Aof6«Lsf] kf]lhl6e (+ve) 6ld{gnnfO{ csf]{ Aof6«Lsf] g]u]l6e (-ve) 6ld{gn;Fu hf]l8G5 . cGTodf, klxnf] Aof6«Lsf] kf]lhl6e 6ld{gn / clGtd Aof6«Lsf] g]u]l6e 6ld{gnnfO{ cfp6k'6sf] (Output) ?kdf lnOG5 .

- Kof/fnn h8fg (Parallel connection) Kof/fnn h8fgdf ;a} Aof6«Lx?sf] kf]lhl6e (+ve) 6ld{gnx? Ps} 7fpFdf hf]l8G5 / ;a} Aof6«Lx?sf] g]u]l6e (-ve) 6ld{gnx? Ps;fy hf]l8G5 . o;k¦sf/sf] Joj:yf Aof6«L a}ª\ssf] Ifdtf a9fpgsf nflu pRr ef]N6]h l:y/ /fVg k¦of]u ul/G5 .

- l;/Lh Kof/fnn h8fg (Series parallel connection) l;/Lh Kof/fnn h8fgsf] ;+of]hg Tolta]nf cfjZos k5{ ha !@ ef]N6eGbf dflysf] ef]N6]h / al9 Ifdtf rflxG5 .

## !=#  OGe6{/ (Inverter)

;f}o{ k|0ffnLdf k|of]u ug{ ;lsg] sltko pks/0fx? ;+rfng ug{ AC kfj/sf] cfjZos kb{5 . OGe6{/n] ;f]nf/ Kofgnaf6 cfPsf] DC kfj/nfO{ AC kfj/df kl/jt{g ug{] ub{5 . ;f] AC kfj/sf] k|of]u ul/ k|of]ustf{x?n] lah'nLsf] nfe lng ;S5g\ . l;+un km]h nf]8sf nflu @#) ef]N6, l;+un km]h OGe6{/ k|of]u ul/G5 / y|L km]h nf]8sf nflu y|L km]h -$)) ef]N6_ OGe6{/ k|of]u ul/G5 .

| - l:6«ª OGe6{/ (String inverter) ljz]if u/L ;fgf] ;f}o{ ?km6k k¦0ffnL h:t} cfjf;Lo 3/x?df o; k¦sf/sf] OGe6{/ k¦of]u ul/G5 . l:6«ª OGe6{/ ;]6ckdf, ;f}o{ Kofgnx¿sf] >[Înf -l:6«ª_ tof/ ul/Psf] x'G5 . ;Dk"0f{ l:6«ªaf6 pTkGg x'g] DC lah'nLnfO{ Pp6f OGe6{/df k7fOG5, h;n] AC lah'nLdf ¿kfGt/0f ub{5 . o;sf] nfut sd x'g'sf] ;fy} ;/n l8hfOgdf x'g] ePsf]n] install ug{ / dd{t ug{ ;lhnf] x'G5 . o;sf] k¦of]udf x'g] d'Vo r'gf}tL eg]sf] s'g} Pp6f Kofgndf 5fFof k/sf] cj:yfdf ;Dk"0f{ l:6«ªsf] cfp6k'6 36fpF5 . |  | lrq ^: l:6«ª OGe6{/ |  |
| --- | --- | --- | --- |
| lrq &: dlN6 l:6ª OGe6{/ | - dlN6 l:6«ª OGe6{/ (Multi string inverter) dlN6 l:6«ª OGe6{/df DoflS;dd kfj/ kf]OG6 6«flsª (Maximum Power Point Tracking , MPPT) Ogk'6x¿ (Inputs) x'G5 . o;n] ;f}o{ P/]nfO{ w]/} l:6«ªx¿df ljefhg ug{ ;lsG5 / k¦To]s l:6«ªnfO{ OGe6{/df 5'§f5'§} ¿kdf h8fg ug{ ldN\5 . |  |
| - ;]G6«n OGe6{/ (Central inverter) ;]G6«n OGe6{/x¿nfO{ 7'nf ;f}o{ lu¦8 h8fg ePsf 7fpFdf k¦of]u ul/G5 . of] OGe6{/, w]/} l:6«ª OGe6{/ h8fg ePsf] h:t} sfd ub{5 . o; OGe6{/df Pp6} P/]x¿nfO{ w]/} l:6«ªx¿ ;dfj]z u/L ljleGg P/]x¿df pk ljeflht ug{ ;lsG5 . |  | lrq *: ;]G6«n OGe6{/ |  |

## !=$  O{ghL{ ld6/ (Energy meter)

;f}o{ ?km6k k|0ffnL h8fg ePsf] ejgdf ljB'tLo nf]8sf] vkt /]s8{ ug{sf] nflu O{ghL{ ld6/sf] k|of]u ug{] ul/G5 . pkef]QmfnfO{ vkt ul/Psf] ljB'tLo o'lg6sf] lan ul/G5 . ljleGg pkef]Qmfx¿ h:t}M cfjf;Lo 3/x¿, pBf]ux¿ tyf Jofj;flos If]qx¿sf] ljB'tLo vkt b/ km/s x'G5 .

;f}o{ ?km6k k|0ffnLdf g]6 ld6l/ª (Net metering) sf] ;fy, afO{-l8/]Szgn ld6/ (Bi-directional meter)--g]6 ld6/_ sf] k|of]u ul/G5 . afO{-l8/]Szgn ld6/dsf] tLg cfwf/e"t sfo{x¿ x'G5 .

- lu|8df lgof{t ul/Psf] pmhf{ dfkg ug]{M ha ;f}o{ PV k|0ffnLn] ejgn] vkt u/]sf] eGbf a9L lah'nL pTkfbg u5{, cltl/Qm lah'nL lu|8df k7fOG5 . afO{-l8/]Szgn ld6/n] of] lgof{t pmhf{ /]s8{ ub{5 .

- lu|8af6 cfoft ul/Psf] pmhf{ dfkg ug]{M ;f}o{ PV k|0ffnLn] cfjZostfeGbf sd lah'nL pTkfbg ug]{ ;dodfM h:t} /ftdf jf afbn ePsf] lbgdf, lu|8af6 lah'nL lnG5 . ld6/n] of] cfofltt pmhf{nfO{ klg 6«ofs (Track) ub{5 .

- pmhf{ k|of]u u0fgf ug]{M afO{-l8/]Szgn ld6/ k|efjsf/L ¿kdf lu|8af6 lgof{t / cfoft ul/Psf] pmhf{sf] 6«ofs ub{5 . lalnª cjlwsf] cGTodf, pkof]lutf sDkgLn] pmhf{ k|of]u u0fgf ug{ of] c+s k|of]u ub{5 .

- olb cfoft ul/Psf] eGbf a9L pmhf{ lgof{t ul/Psf] lyof] eg], pkof]lutf sDkgLn] artsf] nflu k|of]ustf{nfO{ qm]l86 (Credit) lbg ;S5 .

- olb lgof{t ul/Psf] eGbf a9L pmhf{ cfoft ul/Psf] lyof] eg], k|of]ustf{n] lu|8af6 k|of]u ul/Psf] pmhf{sf] nflu e'QmfgL ub{5 .

## !=%  TOD ld6/

| TOD ld6/n] k¦To]s lbgsf] lglZrt ;dosf] pmhf{ vkt /]s8{ ug{] sfd ub{5 . o; ld6/df T1, T2 / T3 tLg lglZrt ;do cg';f/sf] pmh{f vktsf] /sd tf]lsPsf] x'G5 . g]kfndf TOD ld6/ NEA (Nepal Electricity Authority) åf/f cfk"lt{ / h8fg ul/G5 . s'g} klg cfjf;Lo, Jofj;flos jf cf}Bf]lus ;'ljwfdf h8fg ul/Psf] TOD ld6/n] afO{-l8/]Szgn ld6/ (Bi-directional meter) n] ug]{ sfd klg ub{5 . t;y{ TOD ld6/ /flvPsf] cj:yfdf csf]{ afO{-l8/]Szgn ld6/sf] cfjZostf kb{}g . | lrq (: TOD ld6/ |
| --- | --- |

lrq %: sG6«f]n/ aS; / kDk sG6«f]n/

## !=^  cGo ljB'tLo / h8fgsf ;fdfgx¿ 
(Other electrical and installation components)

### !=^=!  ;f}o{ Kofgn / :6«Sr/ (Solar panel and structure)

- ;f}o{ Kofgnx? blIf0ftkm{ kmsf{P/ 5fofF gkg{] u/L /fVg' kb{5 . g]kfnsf] ;Gbe{df ;'o{af6 pmhf{ pTkfbg ug{sf] nflu Kofgnx? @)-#) l8u|Lsf] sf]0fdf 9Nsg] (20-30 degree tilt) u/L /fVg'k5{ .

- 5fofFaf6 hf]ufpgnfO{ hdLgdf pld|g] jg:ktLnfO{ lgoldt ¿kdf sf6\g' k5{ .

- ;a} Kofgnx? g6, af]N6 (Nut and bolt) / jfz/sf] (Washer)] ;xof]un] hf]l8Psf] x'g'kb{5 / lgoldt ¿kdf hfFr ug'{kb{5 .

| lrq !): 5fofF k/]sf] ;f}o{ Kofgnx? | lrq !!: 5fofF gk/]sf] ;f}o{ Kofgnx? |
| --- | --- |

### !=^=@  ljt/0f aS;x¿  (Distribution boxes)

ljt/0f aS;x¿ lah'nLsf] ;'/lIft ljt/0f / lgoGq0fsf] nflu k|of]u x'G5g\ . d'Vo ¿kdf oL aS;x¿n] lah'nLnfO{ ljleGg ;ls{6df k¥'ofpg], k|0ffnLnfO{ cf]e/nf]8, ;6{ ;ls{6, / ;h{af6 arfpg], / dd{t ;lhnf] agfpg] sfo{ u5{g\ . ljt/0f aS;n] ;Dk"0f{ k|0ffnLnfO{ ;'/lIft, Jojl:yt, / dd{t ug{ ;lhnf] 
agfpF5 . ljt/0f aS;sf d'Vo sDkf]g]G6x¿:

- ;ls{6 a|]s/M cf]e/nf]8 jf ;6{ ;ls{6 x'Fbf lah'nL sf6\5 .

- ˆo'hM cf]e/s/G6 x'Fbf ;ls{6 vf]lnlbG5 .

- ;h{ k|f]6]S;g l8efO; -SPD_M lah'nL ;h{af6 ;'/Iff u5{ .

- l/n]M :jrflnt ¿kdf lgoGq0f ug{ d2t u5{ .

- a;af/M lah'nLsf] ;dfg ljt/0fsf nflu .

ljz]iftfx¿

- kfgL / w'nf] k|lt/f]wL -IP65/66_

- clUg k|lt/f]wL

- tfnf nufpg] Joj:yf -Locking mechanism_

;f}o{ ?km6k k|0ffnLdf b'O{ k|sf/sf ljt/0f afs;x? x'G5g\ .

| s) DC ljt/0f aS; (DCDB) |  |  |
| --- | --- | --- |
| lrq 12: DCDB | 8L;L8LaL aS; (DCDB) eg]sf] 8fO/]6 s/]G6 ljt/0f aS; (Direct current distribution box) xf] . of] ;f}o{ Kofgn / OGe6{/ aLr h8fg ul/Psf] x'G5 . 8L;L8LaL aS; (DCDB Box) ;f}o{ k¦0ffnLsf] dxTjk"0f{ efu xf] h;n] ;f]nf/ Kofgnaf6 cfpg] DC kfj/nfO{ ;'/lIft ¿kdf cGo pks/0fx¿df ljt/0f u5{ . of] aS;n] k¦0ffnLnfO{ ;ls{6, kfj/ ;h{, / cGo ljB't ;d:ofaf6 ;'/lIft /fV5 . |  |
| DCDB aS;sf d'Vo efux¿: - DC ˆo'hM cf]e/s/]G6 xF'bf ;ls{6nfO{ ;'/lIft /fV5 . - ;h{ k¦f]6]S;g l8efO; -SPD_M lah'nL rlDsFbf cfpg] ef]N6]h :kfOsaf6 /Iff u5{ . - DC MCB -ldgL ;ls{6 a¦]s/_M cf]e/s/]G6 jf ;6{ ;ls{6 xF'bf ;ls{6nfO{ tf]8\5 (Break) . - a;af/M w]/} sDkf]g]G6sf] laGb'x¿ Ps} 7fpFdf h8fg u/L kfj/ ljt/0f ul/G5 . - cfO;f]n];g l:jrM dd{tsf] nflu kfj/nfO{ l8:sg]S6 ug{ k¦of]u x'G5 . of] aS;n] k¦0ffnLnfO{ ;'/lIft, l:y/ / k¦efjsf/L agfpg d2t u5{ . |  |  |
| v) AC ljt/0f aS; (ACDB) |  |  |
| ACDB eg]sf] cN6/g]l6e s/]G6 ljt/0f aS; (Alternating current distribution box) xf] . of] ;f]nf/ OGe6{/ / nf]8x¿ aLr h8fg ul/Psf] x'G5 . o; leq lgDg efux? x'G5g\ h;sf] sfo{x? tn pNn]v ul/Psf 5g\ . |  |  |
| - AC-SPD: pRr ef]N6]hnfO{ cly{ª u/]/ kfj/ ;h{af6 hf]ufpF5 . - sd ;ls{6 s/]G6 (Low circuit current) sf] nflu AC-MCB (ldlgPr/ ;ls{6 a¦]s/, MCB) jf pRr ;ls{6 s/]G6 (High circuit current) sf] nflu AC-MCCB (df]N8]8 s]; ;ls{6 a¦]s/, MCCB) pRr ef]N6]hsf] cj:yfdf ;ls{6 a¦]s (Circuit break) x'G5 . - a;af/ (Busbar): w]/} sDkf]g]G6sf] laGb'x¿ nflu Ps} 7fpFdf h8fg u/L kfj/ ljt/0f ul/G5 . - cy{af/ (Earth bar): cy{ 6ld{gnsf] h8fgsf] nflu k¦of]u ul/G5 . |  | lrq !#: ACDB |

### !=^=#  c6f]d]l6s 6«fG;km/ l:jr (Automatic Transfer Switch)

c6f]d]l6s 6«fG;km/ l:jr (ATS) n] ljB't cfk"tL{df cj/f]w jf ;d:of b]lvg] lalQs} lah'nLsf] nf]8nfO{ :jrflnt ¿kdf k|fylds ;|f]t (Primary source) af6 Aofsck ;|f]t (Backup source) df kfj/ cfk"lt{     -l:jr_ ub{5 . o;n] lg/Gt/ lah'nL cfk"lt{ lgu/fgL ub}{ nf]8nfO{ Aofsck ;|f]tdf ;xh ¿kdf :yfgfGt/0f ug]{ x'gfn] 8]6f ;]G6/, c:ktfn, cf}Bf]lus k|lti7fg / ;'/Iff k|0ffnLh:tf lg/Gt/ sfo{/t /xg'kg]{ k|0ffnLx¿nfO{ lgaf{w lah'nL cfk"lt{ u/fpg dxTjk"0f{ e"ldsf lgjf{x ub{5 .

To;}u/L, Dofg'cn kl/jt{g jf 6«fG:km/ l:jrn] (Manual changeover or transfer switch) k|of]ust{fx¿nfO{ Dofg'cn ¿kdf k|0ffnLsf] kfj/ ;|f]tnfO{ Ps ;|f]taf6 csf{]df l:jr ug{  d2t ub{5 . 
of] ;fdfGotof Aofsck kfj/ ;|f]t h:t}M h]g]/]6/ jf ;f}o{ pmhf{ h8fg u/L d'Vo kfj/ ;|f]t -h:t} lu|8 kfj/_ gePsf] cj:yfdf l:jr lkm\nk u/]/ k|of]ust{fx¿nfO{ lg/Gt/ lah'nL cfk"lt{ ;'lglZrt ub{5 .

lrq !$: c6f]d]l6s 6«fG;km/ l:jr

### !=^=$  tf/x? (Wires and cables)

- ;a} v'nf tf/x? /fd|/L afFlwPsf] / pko'Qm sG8\o'6 (Conduit) leq x'g'k5{ .

- olb tf/x?df l;w} 3fdsf] k|sfz k5{ eg], cfd{8 s]an (Armoured) jf sG8\o'6 (Conduit) ;lxtsf] o"eL (UV) s]an k|of]u ug{'k5{ .

- ;Dej eP, aflx/L p2]Zosf nflu pko'Qm qm;;]Szgsf] o"eL tf/ (Cross section-UV cable) k|of]u ug'{k5{ .

- ;f}o{ Kofgn k5fl8sf] tf/x?nfO{ /fd|/L afFWg h?/L x'G5 .

| lrq !^: s]an 6fO{ | lrq !&: s]an sG8\o'6 ;lxt |
| --- | --- |

### !=^=%  cly{ª\u÷u|fplG8ª\u lk6 (Earthing/Grounding pit)

- Kofgn, Kofgnsf] :6«Sr/ / sG6«f]n/ a8LnfO{ u|fplG8ª\u (Grounding) ug{'k5{ .

- df6f]sf] ljB't cj/f]w (Resistance) sd ePsf] cj:yfdf u|fplG8ª\u ug'{k5{ .

- u|fpG8 On]S6«f]8x? (Ground electrode) ufl8;s] kl5 hldgsf] cj/f]w dfkg ul/G5 . k|foh;f], u|fplG8ª cj/f]w !) cf]xd (10 Ohm) jf ;f]eGbf sd x'g'k5{ .

# @= On]S6«f]d]sflgsn pks/0fx¿sf] dd{t / ;kmfO{

lgoldt dd{t ;Def/n] ;f}o{ kmf]6f]ef]N6fOs (PV) k|0ffnL w]/} jif{;Dd ;d:of lagf k|of]udf Nofpg ;lsG5 . ;fdfGotof ;f}o{ ¿km6k k|0ffnLnfO{ ;fdfGo dd{t ;Def/sf] cfjZostf k5{, h'g ;lhn} ug{ ;lsG5 . ;f}o{ k|ljlw h8fg ePsf] :yfgdf lgDg cEof;x? ul/g' kb{5 .

## @=!  ;f}o{ Kofgnsf] dd{t / ;kmfO{

### @=!=!  ;f}o{ Kofgnsf] dd{t / ;kmf ug]{ k|lqmof

| l;k | - Kofgn ;kmf ug{] . |
| --- | --- |
| cfjZos ;fdfg | - ;kmf kfgL - ;kmf sk8f - nfdf] Xof08n (Long rod) ePsf] a'¦; (Brush) |

| qm.;. | r/0fx? |
| --- | --- |
| ! | ;f]nf/ kftfsf] cuf8Lsf]] efunfO{ kfgLn] ;kmf ug{], |
| @* | ;kmf sk8f jf a'¦;n] (Brush) kftfnfO{ k'5\g], |
| # | sk8fnfO{ ;kmf u/]/ ;'sfpg], |
| $ | ;'s]sf] sk8f / a'¦;nfO{ prLt :yfgdf e08f/ ug{] . |

* sk8f jf a'|;df s8f a:t'x? k/]df ;f]nf/ kftf sf]l/g ;S5 .

| lrq @): r/0f ! | lrq @!: r/0f @ | lrq @@: ;f}o{ Kofgn ;kmf ub{} |
| --- | --- | --- |

### @=!=@  ;f}o{ Kofgnsf] ef]N6]h / s/]G6 hfFRg] k|lqmof

| l;k | - kftfsf] ef]N6]h / s/]G6 (Voltage and current) gfKg], - 8L;L 8LaL aS; (DC DB Box) hfFRg], - r6\ofË k¦tL/f]wfTds k¦ljlw (LA protection system) hfFRg] . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - Knfo/ (Plier) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |  |  |
| --- | --- | --- | --- |
| ! | dlN6ld6/ tof/ ug{]: - dlN6ld6/ ;]6df /x]sf] sfnf] tf/ jf lkgnfO{ COM port df /fVg]], - dlN6ld6/ ;]6df /x]sf] /ftf] tf/ jf lkgnfO{ Volt port df /fVg]], (b]vfOPsf] tl:j/ cg';f/ tf/ hf]8\g ;lsG5) . | lrq @): dlN6ld6/ |
| @ | dlN6ld6/ DC volt df ;]6 ug{], |  |  |
| # | sfnf] tf/sf] csf]{ 5]psf] lkg (Prob) n] ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] (Junction box) g]u]6Le (-) 6ld{gndf (Terminal) 5'g], |  |  |
| $ | /ftf] tf/sf] csf]{ 5]psf] lkg (Prob) n] ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] (Junction box) kf]h]6Le (+) 6ld{gndf (Terminal) 5'g], |  |  |
| % | dlN6ld6/sf] l8:kn]df (Display) b]vfPsf] ef]N6]h sflkdf gf]6 ug{], |  |  |
| ^ | dlN6ld6/nfO{ Ampere df ;]6 ug{], |  |  |
| & | sfnf] tf/sf] csf]{] 5]psf] lkg (Prob) n] ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] g]u]6Le (-) 6ld{gndf 5'g], |  |  |
| * | /ftf] tf/sf] cs{f] 5]psf] lkg (Prob) n] ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] kf]h]6Le (+) 6ld{gndf df 5'g], |  |  |
| ( | dlN6ld6/sf] l8:kn]df b]vfPsf] s/]G6 sflkdf gf]6 ug{], |  |  |
| !) | dflysf] !-% r/0fsf] k¦lqmof ;a} ;f]nf/ kftfx?sf] hfFr g;s'Gh]n bf]Xof{pg ;lsG5, |  |  |
| !! | dlN6ld6/af6 gfk]sf] ef]N6]h / s/]G6nfO{ Kofgnsf] ljlgb{]zg (Specification) ;+u t'ngf ug{], |  |  |
| !@ | s'g} ;f]nf/ kftfsf] ef]N6]h (Voltage) w]/} g} 36a9 eP ;DalGwt lgsfonfO{ ;'rLt ug{] . |  |  |

### @=!=#  8Ll; 8Lla aS; (DC DB box) hfFRg] k|lqmof

| qm.;. | r/0fx? |
| --- | --- |
| ! | k]rs; k¦of]u ul/ 8Ll; 8Lla aS;sf] 9Ssg vf]Ng], |
| @ | MCB x? cg/ckm (ON/OFF) 5 jf 5}g x]g{], |
| # | s'g} MCB tn em/]sf] 5 eg] dfly ws]nL cg ug{], |
| $ | SPD x?sf] Ol08s]6/ aQL /ftf] jf xl/of] an]sf] 5 x]g{], |
| % | s'g} SPD sf] Ol08s]6/ aQL /ftf] an]sf] 5 eg] gofF km]g{] . |

lrq @!: 8Ll; 8Lla aS; (DC DB Box)

### @=!=$  Surge Protection Device (SPD) km]g{] k|lqmof

| qm.;. | r/0fx? |
| --- | --- |
| ! | 8Ll; 8Lla aS;sf] MCB/MCCB ckm ug{] -tn emfg{]_, |
| @ | df]afO{n k¦of]u ul/ 8Ll; 8Lla aS;sf] kmf]6f] lvRg], |
| # | km]g{ kg{] SPD sf] tf/x? vf]n]/ lemSg], |
| $ | km]g{ kg{] SPD lemSg], |
| % | goFf SPD /fVg], |
| ^ | klxnf lvr]sf] kmf]6f]df h8fg u/] h:t} tf/x? hf]8g], |
| & | hf]8]sf] tf/x? s:g], |
| * | ckm u/]sf MCB/MCCB cg ug{], |
| ( | SPD sf] Ol08s]6/ xl/of] an]sf] 5 eGg] Plsg ug]{, |
| !) | 8Ll; 8Lla aS;sf] 9Ssg aGb ug{] . |

## @=@  Aof6«L :6f]/]h (Battery storage) dd{t / hfFRg] k|lqmof

| l;k | - Aof6«L / Aof6«L 6ld{gnx¿ (Terminals) df w'nf] / lvof hfFr ug{], - Aof6«Lsf] kfgLsf] :t/ (Water-level) hfFr ug{] (cfjZos ePdf), - s'g} klg hf]l8Psf] tf/x¿ v's'nf] jf lalu¦Psf] 5 ls 5}g hfFr ug{], - Aof6«L a}+s (Battery bank) sf] ef]N6]h / s/]06 dfkg ug{], - Aof6«Lsf] :k]l;lkms u¦fle6L (Specific gravity) dfkg ug{], - l8:rfh{ (Discharge) ef]N6]h / s/]G6 dfkg ug{] . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - Sn\ofDk ld6/ (Clamp meter) - k]rs; (Screwdriver) - /]Gr (Wrench) - Knfo/ (Plier) - xfO8«f]dL6/ (Hydrometer) - g/d sk8f (Soft cloth) - k]6«f]lnod h]n (Petroleum jelly) - kGhf (Gloves) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |  |  |
| --- | --- | --- | --- |
| b[li6ut hfFr |  |  |  |
| ! | Aof6«Lx?df w'nf] eP jf gePsf] x]g]{, |  |  |
| @ | Aof6«Lx?sf] 6ld{gndf lvof nfu]sf] 5 jf 5}g x]g]{, |  |  |
| # | Aof6«Lx?sf] kfgLsf] ;txsf] hfrF ug{], |  |  |
| $ | Aof6«Lx? hf]l8Psf] tf/x? l7s 5 ls 5}g cjnf]sg ug{] . |  |  |
| Aof6«Lx?sf] kfgL txsf] hfFr |  |  |  |
| ! | olb Aof6«Ldf /x]sf] kfgLsf] ;tx xl/of] /]vfeGbf tn ePdf - kfgL ;txsf] ;'rsnfO{ lgsfNg], - l8l:6N8 kfgL yKg], - kfgLsf] ;txsf] ;'rsnfO{ aGb ug{], - kfgLsf] ;txsf] ;'rs xl/of] /]vf ;Dd eof] ls ePg eg]/ hfFr ug{], - olb kfgLsf] ;txsf] ;'rs xl/of] /]vfdf 5}g eg] l8l:6N8 kfgL (Distilled water) yKg] . |  |  |
| @* | Aof6«L glhs} HjngzLn (Flammable) j:t'x¿ k¦of]u u/]/ kfgL] txsf] hfFr gug{] . |  |  |
| hf]l8Psf] tf/x¿sf] hfFr |  |  |  |
| ! | Aof6«L;+u hf]l8Psf] MCB nfO{ klxn] aGb ug{] / tf/sf] aflx/L efunfO{ 5'6fpg], |  |  |
| @ | 5'6]sf] jf lau¦]sf] tf/x?nfO{ b'O{ efudf sf6\g], |  |  |
| # | gfËf] tf/x?nfO{ hf]8\g], |  |  |
| $ | - obL sfnf] /+usf] tf/ lau¦]sf]] eP, sfnf] /+usf] 6]k k¦of]u ug{], - obL /ftf] /+usf] tf/ lau¦]sf] eP, /ftf] /+usf] 6]k k¦of]u ug{] . |  |  |
| Aof6«Lsf] ef]N6]h / s/]G6 hfFr |  |  |  |
| ! | dlN6ld6/ tof/ ug{]: - dlN6ld6/ ;]6df /x]sf] sfnf] tf/ jf lkgnfO{ COM port df /fVg]], - dlN6ld6/ ;]6df /x]sf] /ftf] tf/ jf lkgnfO{ Volt port df /fVg]] . |  |  |
| @ | dlN6ld6/ DC volt df ;]6 ug{], |  |  |
| # | sfnf] tf/sf] csf]{ 5]psf] lkg (Prob) nfO{ ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] (Junction box) g]u]6Le (-) 6ld{gndf (Terminal) 5'jfpg], |  |  |
| $ | /ftf] tf/sf] csf]{ 5]psf] lkg (Prob) nfO{ ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] (Junction box) kf]h]6Le (+) 6ld{gndf (Terminal) 5'jfpg], |  |  |
| % | dlN6ld6/sf] l8:kn]df (Display) b]vf7Psf] ef]N6]h sflkdf gf]6 ug{], |  |  |
| ^ | dlN6ld6/nfO{ Ampere df ;]6 ug{], |  |  |
| & | sfnf] tf/sf] csf]{] 5]psf] lkg (Prob) nfO{ ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] g]u]6Le (-) 6ld{gndf 5'jfpg], |  |  |
| * | /ftf] tf/sf] cs{f] 5]psf] lkg (Prob) nfO{ ;f]nf/ kftf d'lg /x]sf] hS;g aS;sf] kf]h]6Le (+) 6ld{gndf df 5'jfpg], |  |  |
| ( | dlN6ld6/sf] l8:kn]df b]vfPsf] s/]G6 sflkdf gf]6 ug{], |  |  |
| !)* | /]s8{ ul/Psf] Aof6«L ef]N\6]h / s/]06 Aof6«L ljlgb{]zx¿df pNn]v ul/Psf] ef]N\6]h / s/]06;+u ldN5 ls ldNb}g t'ngf ug{] . |  |  |
| * glkPsf] Aof6«Lsf] ef]N6]h / s/]06 Aof6«L ljlgb{]zx¿df pNn]v ul/Psf] eGbf sd ePdf ;DalGwt JolQmnfO{ hfgsf/L ug{] . |  |  |  |
| Aof6«Lsf] :k]l;lkms u¦fle6L (Specific gravity) hfrFF |  |  |  |
| ! | xfO8«f]ld6/sf] aNa (Hydrometer bulb) lyRg] , |  | lrq @@: xfO8«f]ld6/ |
| @ | xfO8«f]ld6/sf] l6k (Hydrometer tip) nfO{ On]S6«f]nfO6 (Electrolyte) df 8'afpg], |  |  |
| # | xfO8«f]ld6/sf] aNa (Hydrometer bulb) 5f]8\g], |  |  |
| $ | xfO8«f]ld6/df On]S6«f]nfO6sf] gd"gf (Sample) ;+sng ug]{, |  |  |
| % | xfO8«f]ld6/sf] :s]n x]/]/ :k]l;lkms u¦flj6L lgSof]{n ug]{], - !.@&) – !.#))M cf]e/ rfh{ (Overcharge) - !.@$) – !.@^)M /fd¦f] (Good) - !.!%) – !.@#)M ;fdfGo cj:yf (Normal condition) - !.!)) – !.!$)M k"0f{ l8:rfh{ (Full discharge) |  |  |
| ^ | olb % z]nx? (Cell) !.@$) a/fa/ jf dfly 5 t/ Pp6f z]n !.!)) b]lv !.!%) 5 eg], of] lau¦]sf] x'G5 . |  |  |
| & | k¦To]s z]nsf] :k]l;lkms Uo¦fle6L !.!)) b]lv !.!%), sfod ug{ cfjZos x'G5 . |  |  |
| * | olb z]nsf] :k]l;lkms Uo¦fle6L !.!^) b]lv !.@#) eP o;nfO{ ;fdfGo rfh{ (Normal charge) ePsf] a'lemG5 . |  |  |
| nf]8sf] k/LIf0f |  |  |  |
| ! | d'Vo nfOg (Main grid-line) / ;f}o{sf] MCB/MCCB aGb ug{], |  |  |
| @ | nf]8x?sf] l:jr ON ug{], |  |  |
| # | s]lx ;do;Ddsf] nflu nf]8 ON u/] kl5 Aof6«L ef]N6]h / slt ;dodf ef]N6]h 365, Aof6«L ef]N6]hsf] hfFr ug{], |  |  |
| $ | obL Aof6«Lsf] ef]N6]h sd b]vfPdf, Aof6«Lx?nfO{ abNg' k5{ . |  |  |

ljrf/ ug{'kg{] s'/fx?:

- Aof6«Lx¿ l7s cj:yfdf 5g egL ;'lglZrt ug{] .

- Aof6«L 6ld{gnx¿ lvof d'Qm (Corrosion free) 5g egL ;'lglZrt ug{] .

- Pp6f k|l0ffnLdf Pp6} k|ljlw, lgdf{tf, k|sf/ (Technology, manufacturer, type) sf Aof6«Lx¿ dfq h8fg ug{ l;kmfl/; ul/G5 . obL km/s Aof6«Lx¿ h8fg u/]df rflh{ª / l8:rfh{ (Charging and discharging) ubf{ km/s km/s sfo{Ifdtfsf sf/0f Aof6«Lx¿nfO{ Iflt k'‍{¥ofpg ;S5 .

- Aof6«Lsf] 6ld{gnx¿ ;w}+ v'Nnf x'g] ePsf]n] wft'sf ;dfgx?n] 5'Fbf cfsl:ds ;6{ ;ls{6 x'g ;S5 .

- Aof6«Lx¿df w'nf] / kfgLsf] jfkmaf6 hf]ufpg' kb{5 ;fy} b'O{ Aof6«Lx¿sf] 6ld{gnx?nfO{ Ps cfk;df g5'g] u/L /fVg' kb{5 .

- Aof6«Lx¿ k'5\gsf] nflu kfgLn] leh]sf] sk8f dfq k|of]u ug{'kb{5 . ;'Svf sk8f k|of]u ubf{ On]S6«f]:6]l6s rfh{ (Electrostatic charge) pTkGg eO{ lj:kmf]6sf] x'g] ;+efjgf x'G5 .

- Aof6«L a}+sx¿nfO{ tftf] a:t'sf] glhs jf ;Lw} ;"o{sf] ls/0fsf] ;Dks{ x'g] 7fpFdf /fVg' x'b}g . 
pRr tfkqmdn] ubf{ Aof6«Lsf] l6sfpkg 36\g hfG5 / lj:kmf]6 klg x'g ;S5 .

| Aof6«L 6ld{gn /fd¦f] cj:yfdf ePsf] lvof jf ;n\km]6 km\n]S; (Sulphate flakes) af6 d'Qm ul/Psf] 5 . | 6ld{gndf lvof nfu]sf] h;n] ubf{ Aof6«Lsf] OG6g{n /]lh:6]G; (Internal resistance) a9fpF5 . |
| --- | --- |
| Aof6«L 6ld{gnx¿nfO{ OG;'n]6/n] (Insulated material) ;'/lIft / s]a'nx¿ /fd¦/L h8fg ul/Psf] 5 . | sf8{af]8{sf] k¦of]un] Aof6«L 6ld{gnx¿ 9fSg] u/]sf] 5 . c:yfoL ;dfwfg 6ld{gnx¿ cnu ug{ ko{fKt 5}g . |
| Aof6«L l:6«ªx¿ (Battery strings) zig-zag tl/sfdf / e]lG6n];g (Ventilation) sf] ;fydf k¦ToIf ;"o{sf] hf]lvdaf6 aRg :yfkgf ul/Psf] 5 . | s]xL Aof6«Lx¿ ;Lw} ;"o{sf] ls/0fdf k/]sf] 5 . nfdf] cjlwsf] tfkn] tL Aof6«Lx¿ klxn] lau¦]g] x'g ;S5 . |

## @=#  OGe6{/ hfFRg] k|lqmof

| l;k | - OGe6{/df b[Zo lg/LIf0f hfrF ug{], - OGe6{/sf] ef]N6]h / s/]G6]sf] hfrF ug{], - MC4 sg]S6/ (Connector) sf] hfrF ug{, - OGe6{/df q'l6 (Error code on inverter) hfrF ug{] . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - Knfo/ (Plier) - k]rs; (Screwdriver) - ;kmf sk8f (Clean cloth) - Anf]o]/ of a'?; (Blower or brush) - kGhf (Gloves) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |  |  |  |
| --- | --- | --- | --- | --- |
| b[li6ut hfrF |  |  |  |  |
| ! | OGe6{/df w'nf] 5 ls 5}g x]g{], obL 5 eg] g/d sk8fn] k'5\g], |  |  |  |
| @ | OGe6{/df s'g} ef}lts Iflt (Physical damage) eP gePsf] hfrF ug{], |  |  |  |
| # | OGe6{/af6 v/fa uGw (Bad smell) cfPsf] 5 ls 5}g hfrF ug{], |  |  |  |
| $ | s'g} klg tf/x¿ k'ml:sg nfu]sf] cj:yfdf ePsf] jf efFlrPsf] 5 ls 5}g egL hfrF ug{], |  |  |  |
| % | OGe6{/sf] l8:Kn] (Display unit) df ef]N6]h / s/]G6 slt b]vfPsf] 5 eg]/ x]g{] . |  |  |  |
| OGe6{/nfO{ ;kmf ug{] |  |  |  |  |
| ! | g/d / ;kmf sk8fn] OGe6{/ j/k/ ePsf] kmf]x/nfO{ ;kmf ug{], |  |  |  |
| @ | Anf]j]/ of a'?; (Blower or brush) sf] k¦of]u u/]/ OGe6{/sf] k+vf ;kmf ug{] . |  |  |  |
| OGe6{/sf] efN6]h / s/]G6]sf] hfrF ug{] |  |  |  |  |
| ! | dlN6ld6/sf] k¦of]uaf6 Kofgn / d]g OGk'6 (Panel and main inputs) sf] ef]N6]h hfFRg]], |  |  |  |
| @ | dlN6ld6/sf] k¦of]uaf6 Kofgn / d]g cfp6k'6 (Panel and main outputs) sf] s/]G6 hfFRg] . |  |  |  |
| MC4 sg]S6/ (Connector) hfrF ug]{ |  |  |  |  |
| ! | klxn] OGe6{/sf] MC4 sg]S6/ st} lau¦]sf] 5 jf tftf] 5 ls egL x]g{], |  |  |  |
| @ | obL OGe6{/sf] MC4 sg]S6/ lau¦]sf] jf tfltPsf] ePdf: - DC-MCB jf cfO;f]n]6/ (Isolator) nfO{ aGb ug{], - MC4 sg]S6/nfO{ 5'6fpg], - MC4 sg]S6/sf] tf/nfO{ lgsfNg] clg gFof MC4 sg]S6/ ;f]lx tf/df hf]8\g], - agfPsf] jf gofF MC4 sg]S6/nfO{ OGe6{/df hf]8\g], - DC-MCB jf cfO;f]n]6/ (Isolator) nfO{ ON ug{] . |  |  |  |
| OGe6{/df q'l6 sf]8 (Error code on inverter) hfrF ug{] |  |  |  |  |  |
| ! |  | kfj/ cfp6k'6 gePsf] (No power output) a]nfdf |  |  |  |
|  |  | ;d:of |  | ;dfwfg |  |
|  |  | s. nfdf] ;do ;Ddsf] k¦of]un] ubf{ lvOg] (Wear and tear) jf pTkfbg ubf{sf] cj:yfdf (Manufacturing defects) OGe6{/df cfpg ;Sg] ;d:of, v. Ifltu¦:t jf v's'nf] tf/x¿n], ;f}o{ Kofgnaf6 OGe6{/df lah'nLsf] k¦jfxdf afwf k'/\ofpg ;Sg], |  | s. OGe6{/sf] l8:Kn] (Inverter display) df s]lx q'l6 (Error) b]vfPsf] 5 ls 5}g x]g{], olb OGe6{/df s]lx q'l6 b]vfPdf, dd{tsf nflu k¦of]ustf{ Dofg'cn (User manual) x]g]{ jf ;Dks{ JolQm;Fu ;Dks{ ug{]{], v. ;f}o{ Kofgnaf6 OGe6{/df hf]l8g] tf/x?df s'g} klg Iflt ePsf] 5 jf v's'nf] h8fg ePsf] 5 egL x]g{] . olb s'g} tf/sf] h8fgdf Iflt eP, klxn] DC MCB off ug{] clg tf/nfO{ k"0f{ ?kdf /fd¦f];+u h8fg ug{] . |  |
| @ |  | ;f}o{ OGe6{/ sfd ul//x]sf] t/ sd kfj/ cfp6k'6 (Low power output) ePsf] a]nfdf |  |  |  |
|  |  | ;d:of |  | ;dfwfg |  |
|  |  | s. ¿vx¿, jf ejgx¿ h:tf cj/f]wx¿n] ;f}o{ Kofgnx¿df 5fofF kg{] / k¦0ffnLsf] bIftf 36fpg ;Sg], v. ;f}o{ Kofgnx¿df hd]sf] kmf]xf]/ / w'nf]n] pTkfbg Ifdtf 36fpg ;Sg] . |  | s.;"o{sf] ls/0f cj?4 ug{] cj/f]wx¿ klxrfg u/L x6fpg], v. lgoldt ¿kdf ;f}o{ Kofgnx? ;kmf ug{] . |  |
| # |  | ;f}o{ OGe6{/ cToflws tftf] (Overheating) x'Fbf ;'/Iff ;DaGwL ;d:of / l8:Kn]df ;d:of lgDTofpg ;Sg] x'G5 . |  |  |  |
|  |  | ;d:of |  | ;dfwfg |  |
|  |  | s. k¦efjsf/L ¿kdf kfj/ xfp;sf] tfk sd ug{ kof{Kt e]lG6n];g (Ventilation) gePsf], v. cTolws tftf] df};dn] OGe6{/nfO{ a9L tftf] kfg{ ;Sg] ;+efjgf /x]sf], u. OGe6{/df w'nf] hDdf x'Fbf OGe6{/sf] s"ln+u oGq ;+oGq (Cooling mechanism) df cj/f]w x'g ;Sg] cj:yf ePsf] . |  | s. OGe6{/nfO{ /fd¦/L xfjf rNg] 7fpFdf h8fg ul/Psf] 5 egL ;'lglZrt ug{] / olb ;Dej 5 eg], k+vf h8fg ug{], v. OGe6{/nfO{ k¦ToIf 3fdaf6 hf]ufpg 5fofF kg]{ :yfgdf jf 5f]Kg] u/L /fVg], u. OGe6{/sf] e]G6\; (Inverter’s vent) df hDdf ePsf] s'g} klg w'nf]nfO{ lgoldt ¿kdf ;kmf ug{] . |  |
| $ |  | u¦fplG8df v/faLsf] sf/0fn] ljB'tLo ;'/Iffdf ;d:ofx¿ lgDTofpg] / OGe6{/ aGb x'g ;Sg] (Ground Faults) |  |  |  |
|  |  | ;d:of |  | ;dfwfg |  |
|  |  | s. ljlu¦Psf] jf cgko'Qm ¿kdf h8fg ul/Psf tf/x¿n] u¦fplG8df v/faLx¿ l;h{gf ug{], v. cf]; / lvof (Moisture and corrosion) n] ljB'tLo u¦fplG8df v/faLx¿ cfpg ;Sg] . |  | s. Ifltu¦:t jf v's'nf] tf/x¿sf] h8fgx¿ lg/LIf0f ug,{] v. tf/x¿ / h8fgx¿nfO{ ;'Svf /flv ;'lglZrt u/]/ s'g} klg gdL ;d:ofx¿nfO{ ;Daf]wg ug,{] h+usf] nflu lgoldt ¿kdf hfrF ug{] / s'g} klg h+u ePdf ;f] efunfO{ abn\g] . olb ;d:of /lx/xG5 eg], k"0f{ lg/LIf0fsf] nflu ;Dks{ JolQmnfO{ ;Dks{ ug{] . |  |
| OGe6{/x¿ aLrdf /fd¦f] xfjf ;+rf/ / tftf] lg:sf;gsf] nflu kof{Kt b"/L sfod ePsf] 5 . |  |  | s'g} klg sDkf]g]G6x¿ aLrdf kof{Kt b"/L g/fVbf ;~rfngsf] tfkdfg a9\g]5 . |  |
| lu¦8-OGe6{/ pv k¦0ffnLx¿ k¦ToIf ;"o{sf] 5fofFaf6 ;'/lIft /v]Psf] 5 . |  |  | lu¦8-OGe6{/ pv k¦0ffnLnfO{ l;w} ;"o{ åf/f b\?t ePsf] / cTolws tftf] x'gaf6 /f]sg ko{fKt ¿km yKg] 5 . |  |

## @=$  ACDB dd{t;Def/ ug{]] k|lqmof

| l;k | - ACDB sf] b[Zo lg/LIf0f hfrF ug{], - MCB/MCCB, ELCB, RCB/RCCB sf] sfo{Ifdtf k/LIf0f ug{] . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - Sn\ofDk ld6/ (Clamp meter) - k]rs; (Screwdriver) - g/d sk8f (Soft cloth) - kGhf (Gloves) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| b[li6ut hfrF |  |  |  |  |  |
| ! | ACDB sf] j/k/ w'nf] 5 ls 5}g x]g{], |  |  |  |  |
| @ | klxn] MCB nfO{ OFF ug{], |  |  |  |  |
| # | g/d sk8f k¦of]u u/L w'nf]nfO{ ;kmfO{ ug{] . |  |  |  |  |
| MCB/MCCB sf] sfo{Ifdtf k/LIf0f hfrF ug{] |  |  |  |  |  |
| MCB (Miniature circuit breaker) / MCCB (Moulded case circuit breaker) cf]e/nf]8 / ;6{ ;ls{6 (Overload and short-circuit) af6 ;'/Iff ug{] pks/0f xf] . |  |  |  |  |  |
| ! | MCB / MCCB nfO{ x]/]/ hfrF ug{] h;df hn]sf] jf s]lx Iflt b]v]sf] 5 ls 5}g x]g{], |  |  |  |  |
| @ | MCB / MCCB nfO{ ON / OFF u/]/ h8fg k¦0ffnL ;lemn} rln/x]sf] 5 ls 5}g x]g{], |  |  |  |  |
| # | cf]e/nf]8 6]:6 (Overload test): - a¦]s/ (Breaker) dfkm{t la:tf/} s/]G6 a9fpgsf] nflu nf]8sf] k¦of]u ug{] - a¦]s/ l6«k (Breaker trip) u/]df, o; k¦0ffnLsf] s/]G6 IfdtfeGbf a9L eg]/ yfxf kfpg] - ;DalGwt JolQmnfO{ o; af/] ;Dks{ ug{] |  |  |  |  |
| $ | ;6{ ;ls{6 k/LIf0f (Short-circuit test): - dlN6ld6/nfO{ slG6Go'6L (Contuinuity) ;]l6ªdf /fVg] - ;a} nf]8x¿nfO{ aGb ug{] - dlN6ld6/sf k¦f]ax¿ nfOe (Live, L) / Go"6«n (Neutral, N) tf/sf] k¦of]un] ;6{ ;ls{6 ePsf] 5 ls 5}g x]g{] . olb ;6{ ;ls{6 ePsf] 5 eg], dlN6ld6/n] aLk (Beep) cfjfh u5{ . - nfOe (L) / cy{ (Earth, E) aLr ;fy} Go"6«n (N) / cy{ (E) aLr klg hfFr ug{] tfls oL kyx¿df ;6{ ;ls{6 5}g eGg] ;'lglZrt ug{ ;lsof]];\ . |  |  |  |
| ELCB (Earth Leakage Circuit Breaker) sf] sfo{Ifdtf k/LIf0f ug{] |  |  |  |  |
| sG6«f]n Kofgn (Control panel) df ;6{ ;ls{6 jf cf]e/nf]8 (Short circuit or overload) sf] sf/0fn] x'g] ljB'tLo em6sf / ljB'tLo cfuf] (Electrical shocks and electrical fires) nfO{ /f]Sgsf] nflu ELBC (Earth Leakage Circuit Breakers) sf] k¦of]u ul/G5 . |  |  |  |  |
| ! | ELCB nfO{ x]/]/ hfFr ug{] h;df hn]sf] jf s]lx Iflt b]v]sf] 5 ls 5}g x]g{], |  |  |  |  |
| @ | k/LIf0f a6g (Test button): - cy{ nLs]h q'l6 (Earth leakage fault) hfrF ug{sf] nflu 6]i6 a6g (Test button) nfO{ lyRg], a¦]s/ l6«k (Breaker trip) x'g' k5{ . |  |  |  |  |
| # | cy{ nLs]h k/LIf0f (Earth leakage test): - cy{ nLs]h 6]i6/ (Earth leakage tester) sf] k¦of]u u/]/ nLs]h s/]G6 (Leakage current) yfxf kfpg], cflg ELCB l6«k sf] ;do yfxf kfpg] . |  |  |  |  |
| RCB (Residual Circuit Breaker)/RCCB (Residual Current Circuit Breaker) sf] sfo{Ifdtf k/LIf0f |  |  |  |  |  |
| RCCB n] hldg jf k[YjLsf] q'l6x¿ (Earth faults) af6 hf]ufpF5 / AC / DC b'j}sf] nLs]h s/]G6x¿ (Leakage currents) kQf nufpg ;S5 . |  |  |  |  |  |
| ! | RCCB nfO{ x]/]/ hfrF ug{], hn]sf] jf s]lx Iflt b]v]sf] 5 ls 5}g x]g{], |  |  |  |  |
| @ | k/LIf0f a6g (Test button): - RCCB df /x]sf] 6]:6 a6g (Test button) nfO{ lyr]/ /]l;8\o'cn s/]G6 (Residual current) nfO{ pt]lht (Stimulate) ug{], a¦]s/ l6«k (Breaker trip) x'g'k5{ . |  |  |  |  |
| # | nLs]h s/]G6 k/LIf0f (Leakage current test): - RCCB 6]:6/ (Tester) sf] k¦of]u u/]/ /]l;8\o'cn s/]G6 (Residual current) yfxf kfpg ;lsg], - RCCB df s/]G6 nLs]h (Current leakage) a9L ePdf l6«k ub{5 . |  |  |  |  |

## @=%  ATS (Automatic Transfer Switch) hfFRg] k|lqmof

| l;k | - ;'/Iff pks/0f af/] hfgsf/L, - cly{Ë (Earthing) hfrF ug]{, - nfO{6lgª c/]:6/ (Lightning arrester) hfrF ug]{, - MCBs/MCCBs, SPDs hfrF ug]{ . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - k]rs; (Screwdriver) - cy{ 6]:6/ (Earth tester) - nfO{g] 6]:6/ (Line tester) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |
| --- | --- |
| a¦]s/ l6«k (Breaker trip) sf] hfrF |  |
| ! | kfj/ l:jrx¿ (Power switches) / cf]e/ s/]G6x¿ (Overcurrent) n] a¦]s/nfO{ l6«k ug{] x'gfn] 6«fG;km/ l:jr (Transfer switch) /fd¦/L sfd ub{}g, |
| @ | o; ;d:of nflu ;'bf/ ug{ k¦0ffnLsf] lgoldt dd{t;Def/ ug{' k5{ . |
| cf]; / nLs]h (Moisture and leakage) sf] hfrF |  |
| ! | 6«fG;km/ l:jrnfO{ (Transfer switch) gldsf] jftfj/0faf6 hf]ufpg], |
| @ | s]lx ul/ cf]; / nLs]h (Moisture and leakage) b]lvPdf, ljB'tLo k¦0ffnL (Electrical system) sf] hfrF ug{' clgjfo{ x'G5 . |
| # | g/d sk8f / Anf]j]/ (Blower) sf] k¦of]u ug{], |
| $ | gld / nLs ePsf] ljB'tLo k¦0ffnLnfO{ ;'/Iff pkfox¿ (Safety measures) sf] k¦of]u u/]/ dfq} rnfpg' k5{ . |
| 6«flGhoG6 ef]N6]h (Transient Voltage) sf] hfrF |  |
| ! | ef]N6]h 6«flG;oG6 eg]sf] ljB't k¦jfxdf PSsfl; pRr jf Go"g ef]N6]h b]vf kg]{ cj:yf xf] . o:tf] cj:yf ;fdfGotof klxn] ;+lrt pmhf{ crfgs 5'6\bf (Release), x'bf 7"nf d]l;g jf pks/0fx¿ rnfpFbf, jf ljB't cfk"lt{df cl:y/tf cfpFbf, jf r6\of8Þsf] sf/0f x'g ;S5 . |
| @ | ATS sf] ;'/Iffsf] nflu 6«flG;oG6 ef]N6]h ;h{ k¦f]6]S6/ (Transient voltage surge protector) /fd¦/L sfd u/]sf] x'g'k5{ . |
| ATS sf] dd{t k¦lqmof |  |
| ! | dd{tsf] qmddf d'Vo / Aofsck kfj/ ;¦f]tx¿af6 s'g} klg s/]G6 gcfcf];\ eGgsf nflu b'j}nfO{ ATS af6 l8;sg]S6 ug'{k5{, |
| @ | ATS df s'g} b]lvg] vfnsf] Iflt, h:t} 3if{0f, tftf]sf] c;/, lvof jf lalu¦Psf] 5 ls eg]/ lg/LIf0f ug{], |
| # | ATS sf] aS; /fd¦f] cj:yfdf 5 ls 5}g, / To;df kmf]xf]/, lr;f]kg, jf lvof 5}g eg]/ ;'lglZrt ug{], |
| $ | ;kmf / ;'Svf sk8fsf] k¦of]u u/L ATS sf] leqL efux¿af6 w"nf] / kmf]xf]/ x6fpg], |
| % | 6s{ /]Gr k¦of]u u/]/ ;a} On]lS6«sn sg]Szgx¿, 6ld{gnx¿, / a;af/x¿ s:g] . v's'nf] sg]Szgn] (Loose connection) tfltg] / pks/0f lalu¦g] ;Defjgf x'G5 . |
| ^ | d'Vo kfj/ ;¦f]tdf ljB't cj/f]wsf] l:ylt l;h{gf u/]/, 6«fG;km/ l:jr (Transfer switch) cfjZostf cg';f/ Aofsck kfj/ (Backup power) df ;5{ ls ;b}{g k/LIf0f ug{], |
| & | d'Vo kfj/af6 Aofsckdf / Aofsckaf6 d'Vo kfj/df, lgd{ftfn] l;kmfl/; u/]sf] ;do leq ;5{ ls ;b}{g eg]/ ;'lglZrt ug{], |
| * | dlN6ld6/sf] k¦of]uaf6 ef]N6]h / lk¦mSj]G;L (Frequency) ;xL 5 ls 5}g eg]/ hfrF ug{], |
| ( | ATS sf] u¦fplG8ª k¦0ffnLn] 7Ls;Fu sfd ul//x]sf] 5 / ;a} u¦fplG8ª sg]Szgx¿ ;'/lIft 5g\ eg]/ ;'lglZrt ug{] . |

## @=^  ;'/Iff pks/0f (Protection devices) sf] hfrF

| l;k | - ;'/Iff pks/0f af/] hfgsf/L , - cly{Ë (Earthing) hfrF ug]{, - nfO{6lgª c/]:6/ (Lightning arrester) hfrF ug]{, - MCBs/MCCBs, SPDs hfFr ug]{ . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - cy{ 6]:6/ (Earth tester) - nfO{g-6]:6/ (Line tester) - k]rs; (Screwdriver) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |  |
| --- | --- | --- |
| cly{Ë (Earthing) hfrF ug]{ |  |  |
| ! | cly{Ë u/]sf] :yfg klxrfg ug{], |  |
| @ | cly{Ësf] nflu lk6 -vfN8f]_ k¦of]u ePsf] eP, lk6 vf]Ng], |  |
| # | cly{ËnfO{ cy{ 6]:6/ (Earth tester) k¦of]u h/L hfrF ug]{ tl/sf: - cy{ 6]:6/df ePsf s]annfO{ kfFr / bz dL6/df hf]8\g] ( lrq @# df b]vfOP cg';f/), - cy{ 6]:6/ cg (Earth tester ON) ug{], - cy{ 6]:6/df b]vfPsf] dfkgnfO{ sflkdf n]Vg], - obL dfkg !) cf]xd (10 ohm) eGbf w]/} ePdf, tnsf pkfox? ug]{: - cly{Ësf]] vfN8f]df g'lgnf] kfgL xfNg], - ;do ;dodf cly{Ësf] vfN8f]df kfgL xfNg], |  |
| $ | hfFr u/L ;s]kl5 cly{Ë lk6 aGb ug{] . |  |
| nfO{6lgª c/]:6/ (Lightning arrester) hfrF ug]{ |  |  |
| ! | nfO{6lgª c/]:6/ h8fg u/]sf] 5 ls 5}g x]g{], nfO{6lgª c/]:6/sf] nflu 5'§} cly{Ë x'g'k5{, |  |
| @ | nfO{6lgª c/]:6/sf] nflu lk6 k¦of]u eP, lk6 vf]Ng], |  |
| # | nfO{6lgª c/]:6/sf] cly{ËnfO{ cy{ 6]:6/ k¦of]u h/L hfrF ug]{ tl/sf: - cy{ 6]:6/df ePsf s]annfO{ kfFr / bz dL6/df hf]8\g] (lrq @# df b]vfOP cg';f/ ug{]), - cy{ 6]:6/ cg (Earth tester ON) ug{], - cy{ 6]:6/df b]vfPsf] dfkgnfO{ sflkdf n]Vg], - obL dfkg !) cf]d (10 Ohm) eGbf w]/} ePdf, tnsf pkfox? ug{: - g'lgnf] kfgL cly{Ësf]] vfN8f]df xfNg], - ;do ;dodf kfgL cly{Ësf] vfN8f]df xfNg] . |  |
| lrq @#: cy{ 6]:6/df k¦of]u x'g] /ftf], kx]nf] / xl/of] tf/x? / hfFr ug]{ tl/sf |  |  |
| MCBs/ MCCBs sf] hfrF ug{] |  |  |
| ! | MCBs jf MCCBs h8fg u/]sf] hGS;g aS; (Junction box) nfO{ vf]Ng], |  |
| @ | klxn] MCBs jf MCCBs sf] cj:yf x]g{]: - obL tf/ v's'nf] 5 eg], tf/nfO{ s:g], - obL MCBs jf MCCBs hn]sf] 5 eg], o;nfO{ km]g{], - obL MCBs jf MCCBs glhs w'nf] 5 eg] ;kmf ug{], |  |
| # | dlN6ld6/af6 MCB sf] lg/Gt/tf k/LIf0f (Continuity test) ug{] r/0fx¿: s= ;'/Iffsf nflu ;fjwfgL ckgfpg] - MCB ;Fu hl8t ;a} kfj/ ;¦f]tx¿ aGb ug{'xf];\ . ljB't\ k¦jfx gePsf] kSsf u/]/ dfq sfd ;'¿ ug{'xf];\ tfls s'g} hf]lvd gxf];\ . v= dlN6ld6/nfO{ lg/Gt/tf df]8df ;]6 ug{] - dlN6ld6/nfO{ lg/Gt/tf df]8df ;]6 ug{'xf];\ . of] df]8df k¦foM Wjlg t/+u -;fpG8 j]e_ jf 8fof]8h:tf] lrXg x'G5 . olb lg/Gt/tf df]8 5}g eg], ;a}eGbf sd k¦lt/f]w -Ω_ ;]l6ªdf /fVg'xf];\ . u= dlN6ld6/ k/LIf0f ug{] - MCB k/LIf0f ug{' cl3, dlN6ld6/sf b'j} k¦f]anfO{ ;Fu} hf]8\g'xf];\ . o;n] aLk lbg] jf Go"g k¦lt/f]w b]vfof] eg] o;sf] cy{ dlN6ld6/ 7Ls;Fu sfd ul//x]sf] 5 . 3= k¦f]ax¿nfO{ MCB sf 6ld{gnx¿;Fu h8fg ug{] - Pp6f k¦f]anfO{ Ogk'6 6ld{gndf -hxfF nfOe tf/ h8fg x'G5_ / csf]{ k¦f]anfO{ MCB sf] cfp6k'6 6ld{gndf -hxfF ;ls{6df tf/ h8fg x'G5_ /fVg'xf];\ . ª= l/l8ª (Reading) x]g{] - olb MCB rfn' (ON) cj:yfdf 5 / l7s 5 eg], dlN6ld6/n] aLk lbg]5 -lg/Gt/tf df]8df_ jf sd k¦lt/f]w b]vfpg]5 . o;sf] cy{ lg/Gt/tf 5 . - olb MCB aGb -OFF) cj:yfdf 5 jf lalu¦Psf] 5 eg], dlN6ld6/n] aLk lbg] 5}g jf æOLæ (E) jf pRr k¦lt/f]w b]vfpg]5, h;n] lg/Gt/tf gePsf] hgfpF5 . r= glthf JofVof ug{] - olb rfn' (ON) cj:yfdf lg/Gt/tf b]lvG5 / aGb (OFF) cj:yfdf lg/Gt/tf 5}g eg], MCB 7Ls;Fu sfd ul//x]sf] x'g ;S5 . - olb rfn' (ON) cj:yfdf klg lg/Gt/tf 5}g eg], MCB lalu¦Psf] x'g ;S5 / o;nfO{ kl/jt{g ug{ cfjZos x'G5 . |  |
| $ | clGtddf MCBs jf MCCBs aS;nfO{ aGb ug{] . |  |
| SPDs sf] hfrF ug{] |  |  |
| ! |  | ;'/Iff ;'lglZrt ug{] - hfrF cl3 ljB't\ cfk"lt{ aGb ug{'xf];\ tfls s/]G6 nfUg] hf]lvd gxf];\. OG;'n]6]8 k~hf / cGo ;'/Iff pks/0fx¿sf] k¦of]u ug{] . - 8Ll; 8Lla aS; (DB DC box) sf] MCBs ckm (OFF) ug{] (tn emfg{]),\. - df]afO{n k¦of]u ul/ 8Ll; 8Lla aS;sf] kmf]6f] lvRg] . |
| @ |  | b[Zo lg/LIf0f -leh'cn OG:k]S;g_ - SPD df s'g} zf/Ll/s Iflt, h:t} hn]sf] lrGx, klUnPsf efux¿, jf 9Lnf] ePsf sg]Szgx¿ 5g\ ls 5}gg\ eg]/ hfFr ug{] . - o:tf ;d:of ePdf, pks/0f lalu¦Psf] x'g ;S5 / abNg' cfjZos kg{ ;S5 . |
| # |  | l:ylt ;"rs -:6]6; OlG8s]6/_ hfrF ug{] - w]/} SPD x¿df l:yltsf] cj:yf -:6]6;_ b]vfpg] ;"rs x'G5, h;n] SPD sfo{/t 5 ls 5}g eGg] hfgsf/L lbG5 . - of] k¦foM xl/of] -l7s 5_ jf /ftf] -lalu¦Psf]_ /+usf] x'G5 . - olb ;"rs /ftf] b]vfpF5], SPD n] cfˆgf] Ifdtf u'dfO;s]sf] x'g ;S5 / abNg' k5{ . |
| $ |  | dlN6ld6/åf/f lg/Gt/tf (Continuity) k/LIf0f ug{] - olb dlN6ld6/ pknAw 5 eg], lg/Gt/tf k/LIf0f ug{ k¦of]u ug{ ;lsG5 . - dlN6ld6/nfO{ lg/Gt/tf df]8df ;]6 ug{'xf];\ / SPD sf] Ogk'6 / cfp6k'6 6ld{gnx¿df k¦f]hx¿ /fVg'xf];\ . - olb lg/Gt/tf 5}g eg], of] ;+s]t u5{ ls SPD n] sfd ug{ 5f]8]sf] 5 / abNg cfjZos 5 . |
| % |  | 8So'd]G6];g - lg/LIf0fsf] ;Dk"0f{ glthf b:tfj]h jf 8So'd]G6 ug{'xf];\ . s'g} klg ;d:of, c;fdfGotf, jf lalu¦Psf efux¿sf] af/]df l6Kk0fL n]Vg'xf];\ . dd{t jf abNg] sfd ug{' k/]sf] v08df To;sf] ljj/0f klg ;dfj]z ug{'xf];\ . |
| lalu¦Psf] jf Ifltu¦:t SPD kl/jt{g ug]{ |  |  |
| ! |  | kfj/ ckm ug{] - SPD ;Fu hl8t ;a} kfj/ ;¦f]tx¿ aGb ug{'xf];\, d'Vo / Aofsck b'j}, tfls ;'/lIft ¿kdf sfd ug{ ;lsof];\ . |
| @ |  | lalu¦Psf] SPD x6fpg' - lalu¦Psf SPD af6 tf/x¿ Wofgk"j{s l8;sg]S6 ug{'xf];\ . gofF SPD h8fg ubf{ ;lhnf]sf] nflu tf/x¿sf] h8fgsf] kmf]6f] lvRg'xf];\ . - lalu¦Psf] SPDnfO{ dfplG6ªaf6 ;fjwfgLk"j{s vf]Ng'xf];\ . |
| # |  | gofF SPD h8fg ug{] - gofF SPD nfO{ k'/fgf]sf] :yfgdf /fv]/ dfplG6ªdf sl;nf] u/L h8fg ug{'xf];\ . |
| $ |  | tf/x¿ k'gM h8fg ug{] - lvlrPsf] kmf]6f]cg';f/ gofF SPD df tf/x¿ klxn]s} tl/sfn] h8fg ug{'xf];\ . - ;a} tf/x¿ dha't ¿kdf s:g'xf];\ tfls ;'/Iff ;'lglZrt xf];\ . |
| % |  | MCB cg ug{] - MCB jf SPD ;Fu ;DalGwt a¦]s/x¿ cg -ON_ ug{'xf];\ tfls s/]G6 k¦jfx eP/ SPD sf] sfo{Ifdtf hfFr ug{ ;lsof];\ . |
| ^ |  | SPD Ol08s]6/ aQL hfFr ug{] - SPD sf] Ol08s]6/ aQL xl/of] 5 ls 5}g x]g{'xf];\ . xl/of] aQLn] SPD 7Ls sfd ul//x]sf] ;+s]t u5{ . olb xl/of] 5}g eg], tf/x¿sf] h8fg km]l/ hfFr ug{'xf];\ . |
| & |  | DC DB aS;sf] 9Ssg aGb ug{] - SPD ;xL 9+un] sfd ul//x]sf] k'li6 ePkl5, DC DB aS;sf] 9Ssg ;'/lIft ¿kdf aGb ug{'xf];\ . |

## @=^  TOD ld6/sf] lg/LIf0f

| l;k | - TOD ld6/sf] lg/LIf0f, - ld6/ l/l8ªsf] /]s8{ ug]{ . |
| --- | --- |
| cfjZos ;fdfg | - dlN6ld6/ (Multimeter) - sflk / snd (Notebook and pen) |

| qm.;. | r/0fx? |
| --- | --- |
| ! | ;'/Iff pkfo ;'lglZrt ug{] - lg/LIf0f cl3 ljB't\ cfk"lt{ aGb ug{'xf];\ tfls s/]G6 nfUg] hf]lvd gxf];\ . OG;'n]6]8 k~hf k¦of]u ug{'xf];\ / ;a} ;'/Iff k¦f]6f]snx¿sf] kfngf ug{] . |
| @ | b[Zo lg/LIf0f -leh'cn OG:k]S;g_ - ld6/sf] s];df s'g} zf/Ll/s Iflt, h:t} lr/f, 8]G6, jf v's'nf] efux¿ 5g\ ls 5}gg\ eg]/ hfrF ug{] . - l8:Kn] l:qmg ;kmf / k9\g ldNg] x'g'k5{ . - ld6/sf] jl/kl/ cf];, w'nf], jf kmf]xf]/ hd]sf] 5 ls 5}g eg]/ x]g{] . |
| # | l8:Kn] / /Ll8ª hfrF ug{] - ljB't\ cfk"lt{ rfn' ug{'xf];\ / l8:Kn] ;xL ¿kdf sfd ul//x]sf] 5 ls 5}g eg]/ ;'lglZrt ug{] . - ;do / ldlt l7s 5 ls 5}g k'li6 ug{'xf];\, lsgls TOD ld6/n] ;lx ;dosf cfwf/df sfd u5{ . - l8:Kn]df s'g} c;dfGotf jf cgf}7f] ptf/r9fj b]lvPdf, ;Defljt ;d:of x'g ;Sg] ;+s]t x'g ;S5 . |
| $ | Soflna¦];g / ;]l6ªx¿ hfrF ug{] - TOD ld6/ Soflna¦]6 ul/Psf] 5 ls 5}g / kLs / ckm–kLs ;dodf ;xL ;]6 ul/Psf] 5 ls 5}g x]g{] , - ;do ;]l6ª cg';f/ ld6/n] 7Ls;Fu lks, ckm-lks, / cGo cjlwx¿ aLr :yfgfGt/0f u5{ eg]/ ;'lglZrt ug{], - lks / ckm-lks (Peak and off-peak) cjlwdf /]s8{ ul/Psf] pmh{f vktnfO{ P]ltxfl;s 8]6f jf afXo ;Gbe{ ld6/;Fu t'ngf u/]/ k¦dfl0ft ug{], - TOD ld6/sf] 8]6f / o'l6ln6L (Date and unit) af6 k¦fKt lalnª (Billing) hfgsf/LnfO{ t'ngf ug{'xf];\ / vktsf] l/l8ªdf km/s b]lvPdf of] k¦bz{g ;d:ofsf] ;+s]t x'g ;S5 . - olb Soflna¦];g 6"N;sf] kx'Fr 5 eg], dfkg ;6Ls 5 ls 5}g hfFr ug{ Soflna¦];g k/LIf0f ug{] . |
| % | q'l6 sf]8x¿ hfrF ug{] - ld6/sf] l8:Kn]dfkm{t s'g} klg q'l6 sf]8 jf c;fdfGo l/l8ªx¿ 5 ls eg]/ ;dLIff ug{], - olb ld6/df 8foUgf]l:6s (Diagnostic) ;'ljwf 5 eg], b]vfOPsf] s'g} klg q'l6 sf]8 hfrF ug{'xf];\ . oL sf]8x¿n] ;Defljt uNtL jf Soflna¦];g (Caliberation) ;d:ofx¿ ;+s]t ug{ ;S5g\ . - ld6/nfO{ c;dfg jf lalu¦Psf] kfOPdf, glthfx¿nfO{ o'l6ln6L sDkgL jf pko'Qm dd{t;Def/ ljefunfO{ ;'wf/fTds sbd jf k¦lt:yfkgsf] nflu l/kf]6{ ug{] . |
| ^ | TOD ld6/sf] k¦bz{gnfO{ b:tfj]h jf 8So'd]G6];g ug{'xf];\, h;df s'g} klg c;fdfGotfx¿, ;dosf] leGgtf, ef]N6]h / s/]G6 s'g} c;fdfGo j[l4 jf sdL, jf k/LIf0fsf] qmddf b]vf k/]sf q'l6x¿ ;dfj]z 5g\ . |

## @=&  SRT k|0ffnLdf ck/]6/sf] e"ldsf / lhDd]jf/L
(Operator roles and responsibilities)

SRT k|0ffnLsf ck/]6/n] ;f]nf/ k|0ffnLsf] lgoldt ¿kdf /fd|f];Fu ;~rfng ug{ cg'udg ug]{, dd{t ;Def/ ug]{, / ;d:ofx¿ l/kf]6{ ug{' kb{5 . o;sf] nflu ;f]nf/ sDkgL;+u ;+rf/ u/L k|0ffnLdf ePsf ;d:ofx?sf] hfgsf/L u/fpg] tyf k|fKt lgb]{zg cg'zf/ cfk"mn] jf cGo AolQmx?af6 dd{t ;Def/ ug]{ ug\'{ kb{5 . WhatsApp, Viber, / Team Viewer h:tf ;fdflhs ;~hfnsf] k|of]un] tkfO{FnfO{ sDkgL;Fu ;Dks{df /xg / ;d:ofx¿ l56f] ;dfwfg ug{ ;lhnf] agfpF5 . tn pNn]lvt lgb]{zgx¿ kfngf u/]/, tkfO{Fn] SRT k|0ffnLnfO{ ;xh ¿kdf ;~rfng ug{ ;xof]u k'¥ofpg ;Sg'x'G5 .

s. SRT ck/]6/sf] d'Vo e"ldsfM

  - k|0ffnLsf] lgoldt cg'udg ug]{

- ld6/, Aof6«Lsf] cj:yf, pmhf{ pTkfbg, / kfj/ ljt/0f cflbs] hfFr .

- olb s]lx u8a8 ePsf] cj:yfdf -h:t} sd pmhf{ pTkfbg jf c:jfefljs cfjfh_, t'?Gt pko'Qm sbd rfNg] .

- pks/0fx¿sf] dd{t ;Def/ ug]{

- ;f]nf/ Kofgnx¿, OGe6{/x¿, / cGo pks/0fx¿ lgoldt ;kmf ug]{ tfls ltgLx¿n] /fd|f];Fu sfd u¿g\ .

- n'h sg]Szg jf tf/ Ifltu|:t ePsf] cj:yfdf t'?Gt dd{t ug]{ jf l/kf]6{ ug]{ .

- Aof6«Lx¿ ;xL ¿kdf rfh{ ePsf 5g\ / ltgLx¿nfO{ a9L k|of]u jf Iflt ePsf] 5}g egL ;'lglZrt ug]{ .

- /]s8{ /fVg]

- pmhf{ pTkfbg, Aof6«Lsf] cj:yf, / tkfO{Fn] u/]sf dd{t sfo{x¿sf] b}lgs nu /fVg] .

- s'g} pks/0f lalu|Pdf, To;nfO{ n]Vg] / cfkm]n] ug{ ;Sg] eP To;sf] t'?Gt Joj:yfkg ug]{ jf sDkgLnfO{ hfgsf/L lbg] .

- ;'/Iff k|lqmof kfngf ug]{

- ;f]nf/ k|0ffnLdf sfd ubf{ ;w}F ;xL pks/0f k|of]u ug]{ / ;'/IffTds luo/ -h:t} k~hf, rZdf_ nufpg] .

- olb dd{tsf nflu ;f]nf/ k|0ffnL aGb ug{' k/]df b'3{6gfaf6 aRg ;xL k|lqmof -k|0ffnL aGb ug]{_ kfngf ug]{ .

- ;d:ofx¿ ;dfwfg ug]{

- olb k|0ffnLn] ;xL ¿kdf sfd ul//x]sf] 5}g eg], ck/]6/n] ;d:of kQf nufpg] k|of; ug{'  kb{5 . olb of] ;d:of ;fwf/0f 5 eg] -h:t} tf/x¿ k'gM h8fg ug]{_ cfkm}F ;dfwfg ug]{ / ;d:of uDeL/ 5 eg], sDkgLnfO{ l/kf]6{ ug]{ .

- ;d:of ;dfwfg kl5, k|0ffnLn] km]l/ /fd|f];Fu sfd u/]sf] ;'lglZrt ug]{ .

- u|fxsx¿;Fu cGt/lqmof ug]{

- u|fxsx¿nfO{ ;f]nf/ k|0ffnL cf]e/nf]8 gu/Lsg slt pmhf{ k|of]u ug{ ldN5 eGg] af/]df hfgsf/L lbg] .

- olb s;}nfO{ ljB't cfk"lt{df ;d:of 5 eg], Wofgk"j{s u'gf;fx¿ ;'Gg] / ltgLx¿sf] ;dfwfg ug]{ . olb tkfO{Fn] ;dfwfg ug{ ;Sg'x'Gg eg], sDkgLsf] ;xof]udf ;d:ofnfO{ ;dfwfg ug{ kxn ug]{ .

v. ;~rf/ Pkx¿sf] k|of]u u/]/ k|efjsf/L ;~rf/nfO{ ;xh agfpgM

SRT ck/]6/n] ;f]nf/ k|0ffnLdf cfpg] ;d:of ;dfwfgsf] nflu sDkgL / cGo ;/f]sf/jfnf;Fu ;~rf/ ug{'kb{5 . o;sf] nflu Team Viewer, WhatsApp, Viber h:tf Pkx¿ k|of]u ug{ ;lsG5 .

- WhatsApp jf Viber

oL Pkx¿af6 ;Gb]z, tl:a/ / lel8of]x¿ k7fpg ;lsG5 . o;n] ubf{ ;d:ofx¿ l/kf]6{ ug{, pks/0fx¿df ePs] ;d:ofsf tl:a/x¿ k7fpg, jf online video call ug{ ;lsG5 . pks/0fdf cfPsf] ;d:ofsf] tl:a/ jf lel8of] lvr]/ Pk dfkm{t k7fpg ;lsG5 . o;n] sDkgLnfO{ ;d:of a'‰g ;xof]u ub{5 .

- Team Viewer

of] Pkn] tkfO{Fsf] pks/0fsf] l:qmg c¿;Fu ;femf ug{ ;lsG5 . o;n] ubf{ sDkgLn] online dfkm{t 6f9faf6} kx'Fr u/L ;fO6df guO{sg klg k|0ffnL hfrF ug{ / ;d:of ;dfwfgdf ug{ d2t k'¥ofpF5 .

u. ;~rf/sf] dxTjM

- l56f] ;d:of ;dfwfg ug{M hlt rfF8f] sDkgLnfO{ ;d:ofsf] af/]df hfgsf/L lbg ;lsG5 TotL g} rfF8f] pgLx¿n] ;d:of ;dfwfg ug{ d2t ug{ ;S5 .

- ;xL l/kf]l6{ªM tl:a/ jf lel8of] k7fpFbf, sDkgLn] ;d:ofnfO{ zAbx¿df AofVof u/]eGbf /fd|f];Fu a'‰g ;S5 .

3. k|efjsf/L ;~rf/sf] nflu ;'emfjx¿M

- :k|i6 hfgsf/L lbg]M ;d:of j0f{g ubf{, o;nfO{ ;fwf/0f efiffdf :k|i6 ¿kdf atfpg] / cfjZos k/]sf] cj:yfdf kmf]6f] jf lel8of] ;dfj]z ug]{ .

- rfF8f] ;~rf/ ug]{M ;d:ofx¿ l/kf]6{ ug{ l9nfO ug{' x'b}g . l56f] hfgsf/L lbPdf, ;d:of l56f] ;dfwfg x'g ;S5 / yk IftL x'gaf6 hf]luG5 .

- ;Dks{df /xg]M sDkgLaf6 cfpg] ;Gb]zx¿ yfxf kfpg cfˆgf] kmf]g lgoldt ?kdf x]g]{ .

# #= ;f}o{ ¿km6k nul;6

| sfo{ | ;'wf/fTds pkfox? | b}lgs | dfl;s | q}dfl;s |
| --- | --- | --- | --- | --- |
| Kofgn (Panel) |  |  |  |  |
| Kofgnx¿df w'nf] hfrF ug{] | laxfg jf ;fFemsf] ;dodf kf]5f / sk8fsf] k¦of]u u/]/ Kofgnx¿ ;kmf ug{] . |  |  |  |
| lbpF;f]sf] ;dodf x/]s P/] (Individual array) sf] s/]G6 / ef]N6]h hfrF ug{] | cj?4 ePsf]] 8fof]8 hfrF ug{] . |  |  |  |
|  | olb 8fof]8 lalu¦Psf] cj:yfdf, abNg' cl3 8fof]8sf] kmf]6f] lvRg] . |  |  |  |
|  | 8fof]8 kl/jt{g ug{] / To;sf] kmf]6f] lvRg] . |  |  |  |
|  | olb P/] (Array) ef]N6]h l8hfOg ul/Psf] ef]N6]heGbf sd 5 eg], sDkgLnfO{ ;"lrt ug{] . |  |  |  |
| tf/sf] cj:yf hfrF ug{] | tf/x¿ alnof] agfpg s]an tf/x? k¦of]u ug]{] . |  |  |  |
| glhs}sf] g6-af]N6x¿df lvofsf] hfrF ug{] | lvof nfu]sf] g6-af]N6x¿ ;kmf ug{]] / abNg] . |  |  |  |
| MC4 sg]S6/x¿ hfrF ug{] | olb h8fg v's'nf] ePsf] 5 eg], DC MCBs aGb u/]/ / MC4 sg]S6/x¿ abNg] . |  |  |  |
| P;L / 8L;L sDafOg/ aS; (AC and DC Combiner box) |  |  |  |  |
| sDafOg/ aS; leq w"nf] hfrF ug{] | ;kmf ug{ Anf]c/ (Blower) k¦of]u ug{]] . |  |  |  |
| MCBs sf] Ogk'6 (Input) / cfp6k'6 (Output) ef]N6]h hfrF ug{] | olb ;d:of km]nf k/]sf] cj:yfdf, p:t} d"NofÍg ePsf] ;dfg;+u MCB nfO{ kl/jt{g ug{] . |  |  |  |
| SPDs sf] ;"rs hfrF ug{] | ;"rs /ftf] ePsf] cj:yfdf, SPDs kl/jt{g ug{] . |  |  |  |
| km\o"hsf] lg/Gt/tf hfrF ug{] | olb Ifltu¦:t ePsf] cj:yfdf, km\o"h kl/jt{g ug{] . |  |  |  |
| hfFr ul/Psf] / pmh{f d"n\ofÍgsf] /]s8{ /flvPsf] vkt o'lg6 | olb ld6/n] sfd gu/]sf] cj:yfdf, lgb{]zg ;lxt ld6/ kl/jt{g ug{] . |  |  |  |
| Aof6«L (Battery) |  |  |  |  |
| Aof6«L leq w'nf]sf] l:ylt hfrF ug]{ | g/d / ;kmf sk8fsf] k¦of]u u/]/ w'nf] ;kmf ug{] . |  |  |  |
| Aof6«Ldf lvofsf] l:ylt hfrF ug]{ | tftf] kfgL / a'?zsf] k¦of]u u/L Aof6«L 6ld{gn ;kmf ug{] . |  |  |  |
|  | g6-af]N6 lau¦]df, g6-af]N6 / s]an z' (Cable shoe) kl/jt{g ug{] . |  |  |  |
| xfO8«f]dL6/ k¦of]u u/]/, ;a} Aof6«L ;]nx¿sf] :k]lrlkms u¦lj6L hfrF ug]{ | rflh{ª cj:yfsf] ;dodf, olb :k]l;lkms u¦le6Lsf] n]en sd 5 eg], k¦To]s ^ dlxgfdf OlSjnfOh];g k¦of]u ug{] . gf]6M gr Ö 1.100 -l8:rfh{_, = 1.250 -rflh{ª_ |  |  |  |
|  | olb Aof6«L dWo] Ps df Gr =1.100 5 eg], Aof6«L k¦lt:yfkg ug{ ÷ sDkgLnfO{ ;"lrt ug{ cfjZos 5 . |  |  |  |
|  | OlSjnfOh];g ;]l6ª rfh{ sG6«f]n/ jf OGe6{/df pknAw x'G5 . olb ck/]6/n] ug{ c;dy{ eP, sDkgLnfO{ ;'lrt ug{] . |  |  |  |
| kfgLsf] ;tx ;"rsaf6 Aof6«L leq kfgLsf] l:ylt hfrF ug]{ | kfgLsf] ;tx sd ePsf] cj:yfdf xl/of] lrGx ;Dddf l8l:6N8 kfgL yKg] . |  |  |  |
|  | kfgLsf] n]en hfFr ug{ nfO6/ / cGo HjngzLn j:t'x¿ slxNo} k¦of]u gug]{ . |  |  |  |
| Aof6«Lx? lar, Aof6«L b]lv a;af/ ;Ddsf] tf/ h8fg OG;'n]zg Iflt hfrF ug]{ | olb Iflt cjnf]sg ul/Psf] 5 eg], OG;'n]zg 6]k k¦of]u ug{] . |  |  |  |
|  | olb tf/ efFlrPsf] 5 eg], tf/ h8fg ug{ km]?N; (Ferrules) / 6]k k¦of]u ug{] . |  |  |  |
| Aof6«Lsf] yd{n cj:yf -tftf]_ cjnf]sg ug{ hfrF ug]{ | olb tfkdfg pRr 5 eg], Aof6«L kl/jt{g ug{ cfjZos 5 . |  |  |  |
| Aof6«Lsf] cly{ª 7Ls;Fu ul/Psf] 5 ls 5}g hfrF ug]{ | Aof6«L ;+/rgfsf] cly{ª h8fg ug{] . |  |  |  |
|  | olb Aof6«L ;+/rgfsf] cly{ª 5}g eg], Aof6«LnfO{ slxNo} g5'g] . |  |  |  |
| OGe6{/ (Inverter) |  |  |  |  |
| OGe6{/ j/k/sf] w'nf] hfrF ug]{ | g/d / ;kmf sk8fsf] k¦of]un] w'nf] ;kmf ug{] . |  |  |  |
| OGe6{/x¿sf] Ogk'6 / cfp6k'6sf] h8fg hfrF ug]{ | olb h8fg v's'nf] 5 eg], h8fg s:g] . |  |  |  |
| h8fgdf lvof / hn]sf] hfrF ug]{ | tftf] kfgLsf] k¦of]u u/]/ lvof nfu]sf] If]qdf ;kmf ug{] . |  |  |  |
|  | olb tf/ hn]sf] 5 eg], cfjZos ;fdfgx¿ k¦of]u u/L tf/x¿ kl/jt{g ug{] . |  |  |  |
| OGe6{/df cfjfhsf] ;¦f]t -k+vf_ hfrF ug]{ | olb k+vfaf6 cfjfh cfPsf] eP Anf]c/sf] k¦of]un] ;kmf ug{] jf cfjZos k/] k+vf kl/jt{g ug{] . |  |  |  |
| OGe6{/df tftf] cj:yf hfrF ug]{ | tfk ;¦f]tsf] lg/LIf0f -ˆofg, OGe6{/_ ug{] . |  |  |  |
| Aof6«L sd cj:yf hfrF ug]{ | Aof6«LnfO{ ;lj{l;Ë ug{], cGoyf sDkgLnfO{ hfgsf/L lbg] . |  |  |  |
| cf]e/nf]8÷;6{ ;ls{6 (Overload/short-circuit) cj:yf hfrF ug]{ | ;a} nf]8x¿ 5'6fpg]\ / OGe6{/sf] sfd ug]{ cj:yf hfrF ug{] . |  |  |  |
|  | - olb sfd ub}{ 5 eg]M nf]8 hfrF ug{] / ;d:ofx¿ klxrfg ug{] . - olb sfd ub}{ 5}g eg]M sDkgLnfO{ ;"lrt ug{] . |  |  |  |
| Fan chocks sf] l:ylt hfrF ug]{ | k+vfsf] kmf]xf]/ x6fpg Anf]c/ k¦of]u ug{] . |  |  |  |
| ;f}o{ Kofgn b]lv OGe6{/ ;Ddsf] tf/sf] ef]N6]h hfrF ug]{ | ;f}o{ Kofgn / OGe6{/ aLrsf] tf/sf] q'l6 h8fg klxrfg ug{] . |  |  |  |
|  | q'l6 tf/sf] dd{t jf abNg] . |  |  |  |
| kf]lhl6e (+) b]lv cly{ª 6ld{gnx¿df ef]N6]h hfrF ug]{ | olb l:y/ ef]N6]h dfkg ul/G5 eg], cly{+usf] ;d:of x'G5 . |  |  |  |
| g]u]l6e (-) b]lv cly{ª 6ld{gnx¿df ef]N6]h hfrF ug]{ | olb 36\bf] ef]N6]h dfkg ul/G5 eg], tf/sf] h8fg 7Ls 5 . |  |  |  |
| SPD Kofgn ef]N6]h q'l6 hfrF ug]{ | Kofgn ef]N6]h dfkg ug{'xf];\, olb 7Ls 5 eg] SPD kl/jt{g ug{ cfjZos 5 . |  |  |  |
| Isolator / Disconnector sf] l:ylt hfrF ug]{ | Kofgn ef]N6]h dfkg ug{'xf];\, olb 7Ls 5 eg] Disconnector kl/jt{g ug{ cfjZos 5 . |  |  |  |
| MC4 sg]S6/sf] l:ylt hfrF ug]{ | MC4 sf]+g]R6/ hn]sf] jf klUnPsf] cj:yfdf, sg]S6/ kl/jt{g ug{] . |  |  |  |
| Aof6«L OGe6{/af6 AC mains ef]N6]h hfrF ug]{ | olb AC ef]N6]h 7Ls 5 eg], OGe6{/n] sfd ul//x]sf] 5}g . |  |  |  |
| Balance of system |  |  |  |  |
| cly{ª h8fgx? 7Ls;Fu hf]l8Psf] ;'lglZrt ug]{ | cly{ª tf/ / h8fgx? lg/LIf0f ug{] . |  |  |  |
|  | cly{ªsf] sg]S;gx? pko'Qm / lvofd'Qm x'g'k5{ . |  |  |  |
| ;f}o{ Kofgn :6«Sr/ | af]n\6 / cGo :6«Sr/ s;g] pks/0fx? k¦dfl0ft ug{'xf];\ . Sn\ofDkx¿ s:g'xf];\ / cfjZos cg';f/ s:g] pks/0fx? abNg] . |  |  |  |
| cGo | k¦0ffnLsf] ;xL /]vflrq n]an hfrF ug{'xf];\ / cfjZostf cg';f/ ;'wf/ ug{] . |  |  |  |

| ;f}o{ kftfsf] Log sheet |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ldlt | hfrF ug{] JolQmsf] gfd | P/] ;kmf ul/of] | Kofgnsf] ;+/rgf 7Ls 5/ 5}g | s]anx¿sf] cj:yf 7Ls 5 / 5}g | P/] cfp6k'6 ef]n\6]h hfrF ug{] | P/] cfp6k'6 s/]G6 hfrF ug{] | lvof nfu]sf] 5 / 5}g | s}lkmot |

| Aof6«Lsf] Log sheet |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ldlt | hfrF ug{] JolQmsf] gfd | ;kmf ul/of] | Aof6«L sf] 6ld{gndf k]6«f]lnod h]n\nL (5 / 5}g) | Aof6«L ef]n\6]h | :k]l;lkms u¦lj6L (Specific gravity) |  |  |  |  |  | s}lkmot |
|  |  |  |  |  | ;]n ! | ;]n @ | ;]n # | ;]n $ | ;]n % | ;]n ^ |  |

| OGe6{/sf] Log sheet |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ldlt | hfrF ug{] JolQmsf] gfd | OGe6{/ j/k/ ;kmf 5 ls 5}g | s'g} sL/fx¿ 5 ls 5}g | h8fg 7Ls 5 ls 5}g | OGe6{/ ;~rfng 5 ls 5}g | OGe6{/sf] l8:Kn]df ;a} 7Ls b]vfpm 5 ls 5}g | OGe6{/sf] Kof/fld6/x¿ ;a} 7Ls 5 ls 5}g | s}lkmot |

| ;f}o{ k¦0ffnLsf ;fdfgx¿sf] Log sheet |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ldlt | hfrF ug{] JolQmsf] gfd | ;'/Iff vt/fx¿ 7Ls 5 ls 5}g | s'g} kmf]xf]/ 5 ls 5}g | lah'nL h8fg 7Ls 5 ls 5}g | l:jrx¿, RCCBs / CB ;~rfng | ;'/Iff n]an :ki6 b]V5 ls 5}g | ;a} wft' efu h+u d'Qm 5 ls 5}g | s}lkmot |

lrqsf] ;|f]t: Solar energy introduction architecture system, https://www.slideteam.net/media/catalog/product/cache/1280x720/s/o/solar_energy_introduction_architecture_system_slide01.jpg

lrqsf] ;|f]t: Exide Industries Ltd.

lrqsf] ;|f]t: victron energy BLUE POWER, https://www.victronenergy.com/batteries/

lrqsf] ;|f]t: victron energy BLUE POWER , https://www.victronenergy.com/batteries/lithium-battery-12-8v

lrqsf] ;|f]t: Solar PV Rooftop Training Program, https://www.slideshare.net/slideshow/session-05-grid-connected-inverter/95413645#11

lrqsf] ;|f]t: Solar PV Rooftop Training Program, https://www.slideshare.net/slideshow/session-05-grid-connected-inverter/95413645#11

lrqsf] ;|f]t: Solar PV Rooftop Training Program, https://www.slideshare.net/slideshow/session-05-grid-connected-inverter/95413645#11

lrqsf] ;|f]t: Indiamart, https://5.imimg.com/data5/SE/XL/MY-64360618/electric-meters-500x500.jpg

lrqsf] ;|f]t: solar-off-grid-system-installation-service-500x500.jpeg (500×500)

lrqsf] ;|f]t: United with Israel The Global Movement for Israel TM, Israel Inaugurates its Largest Solar Field Yet | United with Israel

lrqsf] ;|f]t: https://www.indiamart.com/proddetail/solar-aspa-dc-array-junction-box-2852097490712.html

lrqsf] ;|f]t: https://encrypted-tbn1.gstatic.com/images?q=tbn:ANd9GcRXFXLLlIcDETSV6Iak1OTtAgE2CK-dvbht9-eZD-jCzh_HUcaS

lrqsf] ;|f]t: Voltacon Energy SRL Limited, https://voltaconsolar.eu/cdn/shop/products/ats-switch-dual-ac-input-source-63a-output_9120fc17-9c1f-4415-b58a-ad0a2a37fe8d.jpg?v=1710154055

lrqsf] ;|f]t: PVDF-hellermantyton-solar-tie.jpg (1800×1202)

lrqsf] ;|f]t: istockphoto-852397334-612x612_612x.jpg (612×408)

lrqsf] ;|f]t: bobvila, How to Clean Solar Panels: 9 Simple Steps to Maximize Solar Energy Production https://www.bobvila.com/wp-content/uploads/2021/09/How-To-Clean-Solar-Panels-Use-a-Garden-Hose.jpg?w=289&h=217

lrqsf] ;|f]t: Dawnice, How Does Dust Affect Photovoltaic Power Generation, https://www.energydawnice.com/wp-content/uploads/2023/02/blog119.png

lrqsf] ;|f]t: SemiconductorForYou, https://www.semiconductorforu.com/wp-content/uploads/2021/06/digital-multimeter.jpg

lrqsf] ;|f]t: Electrical Wiring School, https://i.ytimg.com/vi/fgmN0oCklcM/maxresdefault.jpg

lrqsf] ;|f]t: Amazon.in, https://encrypted-tbn1.gstatic.com/images?q=tbn:ANd9GcTUwbs7fEaw0RhApJyf4KhYxGItnUrjGcl3KdHgeLLfM-PjI8lx

lrqsf] ;|f]t: SAFEWORK METHOD OF STATEMENT, https://safeworkmethodofstatement.com/testing-commissioning-procedure-for-earthing-system-method-statement/